Optimizing YOLOv8 for Efficient Tomato Recognition in Greenhouse Environments 
Using Drone Imagery

1st Oleg Shovkovyy
Artificial Intelligence & Computer Engineering
CMKL University, Bangkok, Thailand
0000-0002-8080-5479  oshovkov@cmkl.ac.th

2nd Hossein Miri
International School of Engineering
Chulalongkorn University, Bangkok, Thailand
0009-0007-2522-1888  hossein.m@chula.ac.th

Abstract — This study explores the application and fine-tuning of YOLOv8 models for real-time tomato recognition using drone imagery in greenhouse environments, with a focus on practical optimization strategies. Our evaluation of YOLO’s speed, robustness, and adaptability revealed that varying batch sizes and epochs had minimal impact on performance. Notably, the YOLOv8n model matched the performance of the YOLOv8x model while reducing training time by up to 60 times. Further fine-tuning identified the Final Learning Rate (lrf) and dataset annotation quality as critical factors for model performance. Optimizing the lrf and enhancing dataset annotations significantly improved accuracy, underscoring their importance in effective YOLO model deployment. Our results demonstrate YOLOv8’s superiority over YOLOv5, with the optimized YOLOv8n model being ready for deployment in future tomato recognition tasks, paving the way for more efficient agricultural monitoring. This work provides valuable insights into object recognition and offers practical guidance for researchers addressing similar challenges.

Keywords — Artificial Intelligence, Machine Learning, Deep Learning, Computer Vision, Agriculture.

# Introduction

Greenhouses play a crucial role in enabling year-round crop production by providing controlled environments that mitigate external factors, such as weather fluctuations. Despite these advantages, optimizing resource utilization and enhancing crop management efficiency remain significant challenges. Traditional methods of crop monitoring and assessment, such as detecting tomato ripeness, often rely on manual inspection, which is labor-intensive and prone to inconsistencies and delays. The advent of Precision Agriculture has introduced transformative opportunities by integrating advanced technologies, such as drone-mounted cameras and Deep Learning algorithms. This study explores the application of the YOLOv8 model for automating tomato ripeness detection in greenhouse settings. By leveraging real-time drone imagery and Machine Learning, this research aims to streamline agricultural practices, reduce labor costs, and improve the accuracy of crop assessment. The focus on YOLOv8’s speed, robustness, and adaptability underscores its potential to revolutionize greenhouse operations, ensuring timely harvesting and optimal resource allocation.

The motivation behind this work includes:

- Enhancing Efficiency: Automating tomato ripeness assessment reduces labor costs and improves resource allocation.

- Maintaining Quality: Timely harvesting of ripe tomatoes ensures optimal quality and prevents over-ripening and spoilage.

- Sharing Practical Insights: Leveraging real-world experiences to share practical insights can benefit fellow researchers, practitioners, farmers, and stakeholders.

# Material and Methods

## Model Selection

While considering the Computer Vision model for this project, the authors conducted an extensive literature review and identified several models and techniques used for object recognition, including Mask R-CNN, YOLO (You Only Look Once), ConvNets, SVM Pair-based Classifier, HSV, Optical Absorption and Scattering Spectra, and Image Fusion Technique. Each model and technique offers distinct advantages. Ultimately, the authors chose YOLO for tomato recognition in greenhouse environments, driven by several compelling reasons.

Firstly, YOLO’s speed and real-time capabilities offer substantial advantages. YOLO-based detection models process images in a single pass, providing remarkable speed during inference [1]. This efficiency makes it ideal for real-time applications such as tomato detection. Additionally, YOLO’s robustness makes it well-suited for complex environments such as greenhouses, where challenges including fluctuating illumination, leaf occlusion, and overlapping tomatoes arise. YOLO effectively handles these complexities [2]. Furthermore, YOLO models are practical for deployment on resource-constrained devices such as drones and edge devices. Moreover, the effectiveness of YOLO is highlighted by the results in a study by [3], where YOLOv8 achieved an impressive accuracy rate of 99.5%. This exceptional accuracy, coupled with YOLO’s speed and adaptability, made it the optimal choice for tomato recognition in dynamic greenhouse environments, ultimately guiding the authors’ decision.

YOLOv8 offers three types of models: Detection, Segmentation, and Classification, each designed to meet a range of tasks and performance requirements, each varying in size. For object detection, which is the primary focus of this research, the most suitable model is the Detection model, which can recognize different classes of objects and provide a bounding box and class label for each detected object [4].

## Drone

The DJI Mini 4 Pro, employed in this project, stands out as an exceptional tool, ideally suited for tasks demanding precision and adaptability, such as greenhouse tomato ripeness recognition. Its omnidirectional obstacle sensing is crucial for navigating the dense greenhouse environment, enabling the drone to maneuver around obstacles without compromising the mission. Additionally, the drone’s superior imaging capabilities are indispensable for accurately assessing tomato ripeness. Budget considerations and service availability were also key factors in the selection process.

## Hardware

The authors initially opted for Google Colab, a cloud-based interactive development environment. However, technical limitations in its free version — such as unexpected interruptions during training sessions beyond 30-50 epochs and a restricted allotment of free computing units — prompted a transition to a local computer. The local machine was equipped with an Intel Pentium 4400 processor running on a 64-bit architecture at 3.30 GHz. It had 8 GB of DDR4 RAM and utilized an NVIDIA GeForce GT1060 6GB GPU for training YOLO models. The GPU supported CUDA v11.8, cuDNN v11.8, and OpenCV v4.9. Later, this setup was upgraded to an Intel Core i5-7500 @ 3.40 GHz with 16 GB of DDR4 RAM. This upgrade significantly accelerated the training process, reducing the time by more than half in some cases. For instance, training the smallest YOLOv8n (nano) model took about 0.8 hours on the old architecture, but only 0.4 hours on the updated system. In contrast, training the largest YOLOv8x (extra-large) model took approximately 90 hours on the old architecture, whereas it only took 20 hours on the updated system.

# Dataset Construction

In this project, the tomato dataset includes images acquired from the greenhouse of the Ban Fak Huai Community Enterprise Group in Fak Huai District, Aranyaprathet District, Sa Kaeo Province, as well as images from the LaboroTomato dataset specifically curated for tomato detection [5]. The greenhouse images were captured using two different cameras — one mounted on a drone and the other on a mobile phone — resulting in a variety of image conditions due to differing resolutions and qualities. The LaboroTomato dataset was sourced from GitHub [5].

The greenhouse imagery was captured at a distance of 0.5–1.0 meters from the tomato bushes. The drone images had a resolution of 1920x1080, RGB color space, and JPG format, with drone videos recorded at 30 FPS and a resolution of 1280x720 in MP4 format. Mobile phone images were also taken at a resolution of 1920x1080, RGB color space, and JPG format. All images were captured under natural daylight, encompassing challenges such as variations in illumination, occlusion, and overlap.

To maintain consistency, the high-resolution images (3024x4032) from the LaboroTomato dataset were resized to match the resolutions of the drone and mobile phone images. Initially, 1,559 tomato images were selected for training, 232 for testing, and 232 for validation purposes. Specific pre-processing steps, such as normalization, resizing, and augmentation, were performed before annotation to optimize model performance.

## Annotation

Labelled data are essential in training YOLO detection models, requiring the incorporation of class labels and positional coordinates for all ground truth bounding boxes within training images. To streamline this manual and labor-intensive annotation process, the graphical image annotation tool CVAT.AI by [6] was employed to meticulously label all ground truth bounding boxes, with annotation files saved in YOLO v1.1 format. Each tomato is categorized into one of three categories based on its ripeness stage: Ripened (Tomatoes exhibit an entirely red color and are ready to be harvested. They are filled with red color on 90% or more of their surface); Unripened (Tomatoes appear greenish and require additional ripening time. They are filled with red color on 30–89% of their surface); Green (Tomatoes are entirely green or white, occasionally with rare red parts. They are filled with red color on 0–29% of their surface) .

# Model Training

## Settings

The YOLO detection models come in various versions: YOLOv8n, YOLOv8s, YOLOv8m, YOLOv8l, and YOLOv8x. Among these, the YOLOv8n model is the smallest and fastest, while the YOLOv8x model is the largest and most accurate. The authors decided to use the fastest model, YOLOv8n, in the initial phase of training to conserve resources..

The initial training approach involved using default hyperparameters, including setting the Learning Rate (lr0) to 0.01 and the Final Learning Rate (lrf) to 0.01. It was planned to fine-tune these parameters gradually. However, for two critical variables — the number of epochs and batch size — there is a lack of clear default guidelines. For these, [7] emphasizes the role of epochs in achieving optimal performance, recommending a starting point of 300 epochs, while [8] suggests batch sizes of 8, 16, 32, or 64, as these generally yield good results. Larger batch sizes are discouraged due to potential memory constraints and performance degradation. Given this information, the authors opted to explore the impact of adjusting these two variables using the smallest YOLOv8n model for object detection predictability.

## Initial Training

Initially, the dataset underwent sequential training sessions of 20, 30, 50, 100, 200, 300, and 400 epochs on both a local PC and Google Colab, utilizing the smallest recommended batch size of 8 for faster processing. Although early recommendations suggested starting with 300 epochs, the authors opted for fewer epochs at the outset due to constraints such as limited PC power and the use of a free Google Colab account. This approach aimed to evaluate how varying the number of epochs, or the number of times the model passes through the entire training dataset, affected model performance. After assessing outcomes at each stage, it was found that increasing epochs beyond 300 did not lead to significant improvements. Therefore, the authors concluded that 400 epochs were sufficient and chose to finalize training at this point. Model performance was evaluated using probabilities assigned to object detection and values in the ‘Normalized Confusion Matrix’ with the average of both metrics considered to comprehensively assess the model’s effectiveness.

In some training sessions, reaching the desired 300 epochs was not possible in most cases due to the EarlyStopping mechanism, which was set with a ‘patience’ value of 100. This mechanism vigilantly monitored the model’s progress and halted the training process when no significant improvements were detected. In some cases, the model’s progress plateaued after 150 or more epochs, indicating that further extending the training duration would not yield substantial benefits. Therefore, it was decided to set the number of epochs to 100. The results of experimenting with different learning rates and batch sizes at this number of epochs are presented in Table 1.

Experimenting with Different Learning Rates and Batch Size

| # | Hyperparameters |  |  |  | Confusion Matrix Normalized |  |  |  | Model Summary |  |  |  | Total Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | epoch | batch | lr0 | lrf | ripe | unripe | green | average | ripe | unripe | green | average |  |
| 1 | 100 | 64 | 0.001 | 0.001 | 89 | 64 | 82 | 78.3 | 88 | 62 | 74 | 74.7 | 76.5 |
| 2 | 100 | 32 | 0.001 | 0.001 | 85 | 66 | 78 | 76.3 | 87 | 56 | 74 | 72.3 | 74.3 |
| 3 | 100 | 16 | 0.001 | 0.001 | 84 | 66 | 85 | 78.3 | 81 | 48 | 66 | 65.0 | 71.7 |
| 4 | 100 | 8 | 0.001 | 0.001 | 84 | 82 | 84 | 83.3 | 87 | 49 | 71 | 69.0 | 76.2 |
| 5 | 100 | 8 | 0.001 | 0.01 | 82 | 70 | 86 | 79.3 | 81 | 44 | 64 | 63.0 | 71.2 |
| 6 | 100 | 8 | 0.001 | 0.1 | 81 | 74 | 89 | 81.3 | 87 | 50 | 68 | 68.3 | 74.8 |
| 7 | 100 | 8 | 0.01 | 0.001 | 84 | 82 | 84 | 83.3 | 87 | 49 | 71 | 69.0 | 76.2 |
| 8 | 100 | 8 | 0.1 | 0.001 | 84 | 82 | 84 | 83.3 | 87 | 49 | 71 | 69 | 76.2 |

The dataset parameters for Table 1 are as follows: 1,559 training images, 323 validation images, and an image size of 640 pixels .

The first observation in the initial training is that increasing the batch size does not necessarily improve the probability of detection in each category; in fact, smaller batch sizes (particularly for this dataset) appear to yield better overall model performance. This observation is supported by the data in rows 4, 7, and 8 of the ‘Normalized Confusion Matrix’ and ‘Total Average’ columns, where training with a batch size of 8 demonstrates superior performance compared to batch sizes of 16, 32, and 64.

| # | Model Type | Dataset |  |  | Hyperparameters |  |  |  | Confusion Matrix Normalized |  |  |  | Model Summary |  |  |  | Model Specs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | training images # | validation images # | image size | epoch | batch | lr0 | lrf | ripe | unripe | green | average | ripe | unripe | green | average | time, hr | size, Mb |
| 1 | YOLOv8n | 1559 | 232 | 640 | 50 | 8 | 0.01 | 0.01 | 81 | 81 | 89 | 83.7 | 85 | 48 | 71 | 67.9 | 0.8 | 6 |
| 2 | YOLOv8n | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 96 | 88 | 87 | 90.3 | 90 | 87 | 96 | 91.0 | 0.9 | 6 |
| 3 | YOLOv8s | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 93 | 92 | 90 | 91.7 | 92 | 84 | 95 | 90.2 | 3.5 | 21 |
| 4 | YOLOv8m | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 89 | 89 | 89 | 89.0 | 95 | 92 | 97 | 94.5 | 8.9 | 52 |
| 5 | YOLOv8l | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 88 | 87 | 86 | 87.0 | 88 | 86 | 96 | 90.1 | 23.0 | 85 |
| 6 | YOLOv8x | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 92 | 81 | 87 | 86.7 | 94 | 94 | 93 | 93.7 | 20.0 | 136 |
| 7 | YOLOv5 | 1122 | 136 | 640 | 50 | 8 | 0.01 | 0.01 | 84 | 74 | 81 | 79.7 | 78 | 85 | 94 | 85.7 | 1.8 | 14 |

The next striking observation is the low probabilities of detection for the ‘green’ and ‘unripe’ categories, in comparison to the ‘ripe’ category, as seen from the ‘Model Summary (Box P)’ columns, suggesting that there could be a problem related to how the model distinguishes between these categories. The issue might lie in the way the images were annotated, making it difficult for the model to distinguish ‘green’ from ‘unripe.’ Consequently, it was concluded that a thorough revision of the annotation methodology was necessary to ensure greater accuracy and reliability in the dataset, which would ultimately enhance the overall performance and effectiveness of the model.

Another observation during the analysis of data in Table 1 reveals that adjustments in learning rates lr0 and lrf have varied, and sometimes conflicting, impacts on prediction accuracy across different object categories. The data displayed in the ‘Normalized Confusion Matrix’ and ‘Model Summary’ indicate that achieving optimal and consistent prediction rates within categories and between these two metrics can pose significant challenges and, in some cases, may be unattainable.

To conclude, a batch size of 8, as evidenced by results presented in Table 1, appears to be sufficiently suitable for future training sessions. As for the number of epochs, it was observed that after approximately 30 epochs, no significant improvements in the model’s performance were detected, challenging the conventional belief in the necessity of lengthy training sessions. Therefore, to balance training time and performance, a deliberate decision was made to fix the following training sessions at 50 epochs. This choice allowed the model to learn effectively without risking overfitting or wasting computational resources. By maintaining this equilibrium, ample learning opportunities were ensured.

## Experimentation

As suggested by the results from the initial training stage, the original dataset was re-annotated. Ambiguous, unclear, and blurred images, as well as those depicting damaged (rotten) tomatoes, were excluded, resulting in a final dataset of 1,122 images for training. Another 136 images (12%) were dedicated to validation, with an equal number reserved for testing purposes.

Rerformance Results on Reannotated Dataset

Training the same YOLOv8n model with identical hyperparameters on this updated dataset produced significantly better probabilities of object detection, as evidenced by the results in Table 2, rows 1 and 2. The detection probabilities for all classes of tomatoes improved, with a particularly notable increase in the ‘unripe’ category. This enhancement substantiates the authors’ initial conclusion regarding annotation issues in the original dataset. It underscores the imperative of a comprehensive revision of the annotation methodology to ensure greater accuracy and reliability, thereby ultimately improving the model’s overall performance and effectiveness.

Encouraged by the model’s improved performance, the updated dataset was used to train other versions of the YOLOv8 Detection models: YOLOv8s, YOLOv8m, YOLOv8l, and YOLOv8x. The goal was to identify which model might be best for final tuning and to determine if there were significant performance differences among these models when tested on the same dataset. Out of a curiosity-driven inquiry, the authors decided to evaluate the dataset using YOLOv5 to assess any significant performance differences. Previous research had demonstrated promising results with YOLOv5 in similar contexts, prompting the authors to compare it against YOLOv8’s newer, more lightweight framework and user-friendly programming. Although YOLOv8 was initially selected for its advanced features, the authors remained open to re-consideration if YOLOv5 proved superior on the same dataset. The objective was to achieve a recognition rate exceeding 93%. However, when YOLOv8 did not meet these expectations, the authors prudently opted to test YOLOv5, considering a potential switch if it offered better performance. The results, however, revealed that YOLOv5 underperformed in comparison to YOLOv8, affirming the soundness of the initial choice. Subsequently, every version of YOLOv8 — nano, small, medium, large, and extra-large — demonstrated superior predictability for tomato classification across all categories, consistently outperforming YOLOv5. This uniformity across YOLOv8 versions underscores the enhanced accuracy and reliability of the updated YOLO framework in object detection tasks.

Furthermore, the YOLOv8 model offers a distinct advantage over YOLOv5 due to its significantly simplified programming process. When comparing the two, several key differences emerge, particularly in the areas of installation, model loading, training, validation, prediction, and exporting. YOLOv8 streamlines installation by utilizing the ultralytics package, which can be effortlessly installed with a single pip command. In contrast, YOLOv5 requires cloning the repository and installing dependencies via a requirements file, making the setup process more complex. Moreover, loading models is notably more streamlined in YOLOv8, where the YOLO class enables direct access to pre-trained models. In contrast, YOLOv5 relies on PyTorch Hub, necessitating additional steps for model loading. The training process further distinguishes the two: YOLOv8 simplifies training through a method call on the model object, seamlessly integrating it within the Python API. YOLOv5, however, requires executing a separate training script with multiple command-line arguments, adding layers of complexity. For validation and prediction, YOLOv8 maintains an integrated approach, managing these tasks directly through the model object. YOLOv5, by comparison, necessitates running distinct scripts for validation and detection. Finally, while both models support exporting to ONNX format, the commands differ slightly. YOLOv8’s method aligns with its overall streamlined design, whereas YOLOv5 requires specific script-based commands.

Overall, YOLOv8 offers a more user-friendly and integrated programming experience, streamlining the development process and making it an appealing choice for developers and researchers seeking simplicity and efficiency. This underscores that selecting YOLOv8 over YOLOv5 was indeed the right decision for this project. Notably, the total average predictions — derived from the ‘Confusion Matrix’ and ‘Model Summary’ columns — were remarkably consistent across the different YOLOv8 Detection models (YOLOv8n, YOLOv8s, YOLOv8m, YOLOv8l, and YOLOv8x). However, the YOLOv8m model stood out by achieving an average object detection probability of 91.8%, the highest among them, while the YOLOv8l model had the lowest with 88.5%. These variations in accuracy can be attributed to differences in the models’ architectures, reflecting the trade-offs between complexity and computational efficiency. The YOLOv8n (nano) and YOLOv8s (small) models are optimized for speed, making them ideal for environments with limited computational resources, albeit with slightly lower accuracy compared to larger models. In contrast, the YOLOv8m (medium) model strikes a balance between speed and accuracy by incorporating more layers and parameters, thereby enhancing its object detection capabilities. Meanwhile, the YOLOv8l (large) and YOLOv8x (extra-large) models, despite their increased layers and parameters, may experience diminishing returns in accuracy due to potential overfitting or the demands of greater computational complexity.

Addressing the analysis, it is important to highlight that YOLOv8s was the sole model to achieve probabilities exceeding 90% across all three categories in the ‘Confusion Matrix.’ Taking into account all relevant factors — especially processing speed, final model size, and the consistency of training predictability — the decision was made that larger models are not necessary for further development. The smaller models present a more efficient balance between performance and resource utilization, making them better suited for ongoing work in drone object detection. As a result, YOLOv8n was chosen for further refinement.

In summary, re-annotating the initial dataset markedly enhanced the performance of the YOLOv8 detection models, especially in identifying distinct tomato categories. This updated dataset surpassed the renowned Laboro Tomato dataset, validating its effectiveness for subsequent training and refinement. While YOLOv8m demonstrated the highest detection accuracy, YOLOv8n was selected for its optimal balance of performance and efficiency. The decision to forgo training the resource-intensive YOLOv8x model was driven by practical considerations. Overall, the results underscore the significance of high-quality annotations and affirm YOLOv8’s superiority over YOLOv5, laying a solid groundwork for future drone-based object detection projects.

## Tuning

This stage involved making careful adjustments to the model’s hyperparameters, particularly to the learning rate (lr0) and the learning rate factor (lrf). By fixing one value and making significant adjustments to the other, the authors observed the effects of these changes on the model’s performance. Concurrently, the batch size and number of epochs were kept the same as in the previous experimentation stage, with the number of epochs set to 50 and the batch size to 8. The goal was to find a balance that maximized the model’s performance, as indicated by the ‘Confusion Matrix’ and ‘Model Summary’ metrics. By carefully optimizing these hyperparameters, the authors aimed to enhance the model’s accuracy and efficiency, ultimately achieving superior object detection performance. Table 3 showcases how this process looks, exemplifying the manipulation of the lrf value. The dataset parameters for Table 3 are as follows: 1,122 training images, 136 validation images, and an image size of 640 pixels.

| # | Hyperparameters |  |  |  | Confusion Matrix Normalized |  |  |  | Model Summary |  |  |  | Total Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | epoch | batch | lr0 | lrf | ripe | unripe | green | average | ripe | unripe | green | average |  |
| 1 | 50 | 8 | 0.01 | 0.0001 | 92 | 85 | 87 | 88.0 | 93 | 83 | 93 | 89.4 | 88.7 |
| 2 | 50 | 8 | 0.01 | 0.0005 | 93 | 86 | 87 | 88.7 | 91 | 92 | 95 | 92.3 | 90.5 |
| 3 | 50 | 8 | 0.01 | 0.0075 | 94 | 92 | 87 | 91.0 | 91 | 90 | 96 | 92.3 | 91.6 |
| 4 | 50 | 8 | 0.01 | 0.0008 | 87 | 89 | 76 | 84.0 | 87 | 92 | 94 | 90.7 | 87.4 |
| 5 | 50 | 8 | 0.01 | 0.001 | 95 | 89 | 87 | 90.3 | 93 | 91 | 97 | 93.4 | 91.9 |
| 6 | 50 | 8 | 0.01 | 0.01 | 96 | 88 | 87 | 90.3 | 90 | 87 | 96 | 91.0 | 90.7 |
| 7 | 50 | 8 | 0.01 | 0.025 | 94 | 86 | 88 | 89.3 | 93 | 89 | 96 | 92.4 | 90.9 |
| 8 | 50 | 8 | 0.01 | 0.05 | 92 | 86 | 86 | 88.0 | 91 | 90 | 94 | 91.7 | 89.9 |
| 9 | 50 | 8 | 0.01 | 0.075 | 96 | 85 | 87 | 89.3 | 90 | 86 | 97 | 91.3 | 90.3 |
| 10 | 50 | 8 | 0.01 | 0.1 | 92 | 86 | 85 | 87.7 | 89 | 90 | 96 | 91.6 | 89.6 |

Manipulating Learning Rate Factor

To achieve the highest average probability of recognition, it was necessary to set the initial learning rate (lr0) to its recommended default value of 0.01 and adjust the learning rate factor (lrf) to a substantially lower value of 0.001. This configuration adheres to the general guideline that the learning rate factor (lrf) for the last layer in YOLOv8 should be significantly smaller than the initial learning rate (lr0), with a common practice being to set lrf to approximately one-tenth of lr0. However, further adjustments to lrf led to the discovery of a new optimal setting. As illustrated in row 3 of Table 3, a specific combination of lr0 and lrf resulted in detection rates exceeding 90% for 5 out of 6 categories.

Given this promising result, the authors opted to set the learning rate factor (lrf) at 0.0075, as the results indicated. They then directed their efforts toward refining the model by adjusting the initial learning rate (lr0). However, further modifications to lr0 did not enhance the prediction rates. The attempt to retest the YOLOv8n model with fixed learning rates (lr0 and lrf) for different batch sizes (16, 32, and 64), as well as with varying numbers of epochs (up to 300), did not result in any improved performance. This further substantiates the findings of this study, emphasizing that the model’s high prediction accuracy does not solely stem from extensive training or the use of a large batch size. With lr0 = 0.01 and lrf = 0.0075 proving effective for the YOLOv8n model the authors were curious to see if these values would yield similar results for the YOLOv8s model. Table 2 shows that the YOLOv8s model produced even better outcomes with default settings (lr0 = 0.01 and lrf = 0.01). However, testing the YOLOv8s model with lr0 and lrf set at 0.01 and 0.0075 did not improve prediction rates; in fact, it worsened them.

## Innovative Approach

To optimize the efficiency and effectiveness of YOLOv8n model training, an innovative approach was implemented to simulate extended training periods. Normally, models are trained sequentially for a set number of epochs, evaluated, and then retrained from scratch for additional epochs. However, this process was refined to achieve the equivalent of training for 100, 150, 200, and 300 epochs in a more systematic and progressive manner while saving training time. First, an initial training session for 100 epochs was conducted, saving the best checkpoint from this stage as best_100.pt. This checkpoint represents the model’s optimized state after 100 epochs of training. Rather than starting a new session from scratch for the subsequent 150 epochs, the continuation was done from best_100.pt. For the second phase, the model was trained for an additional 50 epochs using best_100.pt as the starting point. This method allowed for a seamless transition and a more efficient learning process, resulting in the updated checkpoint best_100.pt. The same approach was followed for the next session of 200 and 300 epochs. This approach not only ensured that the model benefited from cumulative learning but also simulated a continuous 300-epoch training process without starting from scratch at each stage.

By adopting this innovative training strategy, incremental improvements were harnessed while maintaining stability and robustness in the YOLOv8n model. This method saved computational resources and time, as well as optimized the model’s learning trajectory, resulting in higher accuracy and better performance in object detection tasks.

## Further Tuning

In addition to the commonly tuned hyperparameters (such as learning rate, batch size, and epochs), other relevant settings that can impact YOLOv8 model training outcomes were tested. These include manipulating Momentum (β1) rates (a parameter affecting the update of model weights during optimization) within the recommended range of 0.9 to 0.9999; adjusting Weight Decay (L2 Regularization that helps prevent overfitting by adding a penalty term to the loss function) within the range of 0.0003 to 0.00099; and trying alternative optimizers (such as ‘SGD,’ ‘AdamW,’ or ‘RMSProp’).

The results from training YOLOv8n with these additional parameter adjustments, as detailed in Table 4 (the dataset parameters for Table 4 are as follows: 1122 training images, 136 validation images, and an image size of 640 pixels), indicate that none of these modifications improved upon the detection rates achieved by the original YOLOv8s model with default settings and slight modifications to lrf , batch size set at 8, and 50 epochs, presented in Table 3, row 3.

# Conclusion

This study highlights the significant potential of deep learning, particularly YOLOv8 models, in Precision Agriculture within greenhouse environments. Notably, the YOLOv8n variant, despite its compact size, delivers performance comparable to larger models like YOLOv8x, while significantly reducing training times — an essential advantage for real-time applications. Key findings emphasize the critical impact of hyperparameter tuning, especially the Final Learning Rate (lrf) and the quality of dataset annotations on model accuracy.

Manipulating Extra Parameters

| # | Tuned Parameter | Hyper parameters |  |  |  | Confusion Matrix Normalized |  |  |  | Model Summary |  |  |  | Total Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | epoch | batch | lr0 | lrf | ripe | unripe | green | average | ripe | unripe | green | average |  |
| 1 | Default set. | 50 | 8 | 0.01 | 0.0075 | 94 | 92 | 87 | 91.0 | 91 | 90 | 96 | 92.3 | 91.6 |
| 2 | SGD | 50 | 8 | 0.01 | 0.0075 | 90 | 94 | 87 | 90.3 | 94 | 89 | 96 | 93.1 | 91.7 |
| 3 | AdamW | 50 | 8 | 0.01 | 0.0075 | 89 | 94 | 85 | 89.3 | 93 | 87 | 98 | 92.9 | 91.1 |
| 4 | RMSProp | 50 | 8 | 0.01 | 0.0075 | 79 | 67 | 49 | 65.0 | 75 | 73 | 81 | 76.3 | 70.7 |
| 5 | decay-0.0003 | 50 | 8 | 0.01 | 0.0075 | 94 | 86 | 85 | 88.3 | 90 | 85 | 96 | 90.4 | 89.4 |
| 6 | decay-0.0007 | 50 | 8 | 0.01 | 0.0075 | 93 | 88 | 88 | 89.7 | 90 | 88 | 95 | 90.9 | 90.3 |
| 7 | decay-0.0009 | 50 | 8 | 0.01 | 0.0075 | 92 | 91 | 85 | 89.3 | 93 | 88 | 97 | 92.3 | 90.8 |
| 8 | moment--0.95 | 50 | 8 | 0.01 | 0.0075 | 92 | 89 | 81 | 87.3 | 87 | 87 | 96 | 90.0 | 88.7 |
| 9 | moment-0.975 | 50 | 8 | 0.01 | 0.0075 | 90 | 84 | 85 | 86.3 | 90 | 83 | 93 | 88.7 | 87.5 |
| 10 | moment-0.99991 | 50 | 8 | 0.01 | 0.0075 | 81 | 67 | 68 | 72.0 | 91 | 82 | 93 | 88.7 | 80.4 |

The study also underscores the challenge of consistent dataset annotation, particularly for subjective criteria such as tomato ripeness. Variability in ripeness perception among annotators indicates the need for standardized annotation protocols or specialized training datasets tailored to specific tomato varieties and growing conditions. By stressing the importance of meticulous dataset curation and high-quality annotations, this research paves the way for more reliable model performance.

Furthermore, recent research by [9] explored YOLOv8’s performance in multi-crop classification using Unmanned Aerial Vehicle (UAV) images, highlighting the advantages of hyperparameter optimization, particularly in terms of batch size. The authors observed that YOLOv8 achieved notable gains in accuracy and efficiency with an optimal batch size of 60. In comparison, our study employed a batch size of 50, demonstrating steady improvements in model performance across a range of agricultural applications.

Future research should aim to encompass larger datasets that represent a wide range of tomato cultivars and environmental conditions. Collaboration with agricultural institutions, universities, and government agencies, which often maintain extensive databases on crop varieties and environmental factors, will be essential. These organizations frequently possess both historical data and ongoing research initiatives, offering valuable insights. Furthermore, partnering with international agricultural research centers such as the International Tomato Genome Sequencing Project, could provide access to global datasets, enabling comparative studies across different regions. Conducting field experiments and surveys to gather primary data will also be crucial.

Moreover, advanced techniques in model optimization and deployment on resource-constrained devices offer the potential to significantly enhance the practical applications of automated crop monitoring systems. By integrating Precision Agriculture technologies, agricultural productivity can be optimized, resource wastage minimized, and yield efficiency maximized, ultimately contributing to more sustainable farming practices.

##### References

Zheng, S., Liu, Y., Weng, W., Jia, X., Yu, S., & Wu, Z. (2023). Tomato Recognition and Localization Method Based on Improved YOLOv5n-seg Model and Binocular Stereo Vision. Agronomy, 13(9). https://doi.org/10.3390/agronomy13092339.

Liu, G., Nouaze, J. C., Touko Mbouembe, P. L., & Kim, J. H. (2020). YOLO-Tomato: A Robust Algorithm for Tomato Detection Based on YOLOv3. Sensors, 20(7). https://doi.org/10.3390/s20072145.

Xiao, B., Nguyen, M., & Yan, W. Q. (2023). Fruit Ripeness Identification Using YOLOv8 Model. Multimedia Tools and Applications. https://doi.org/10.1007/s11042-023-16570-9.

Hansen, U. S. (2023). Object Detection: Models, Use Cases, Examples. Retrieved from https://encord.com/blog/object-detection/

LaboroAI. (2024). Laboro Tomato: Instance Segmentation Dataset. GitHub. Retrieved 14 March from https://github.com/laboroai/LaboroTomato.

CVAT. (2024). Open Data Annotation Platform. Retrieved March from https://app.cvat.ai.

Jocher, G., & Burhan-Q. (2024). Tips for Best Training Results. Ultralytics Inc. Retrieved 25 June from https://docs.ultralytics.com/YOLOv5/tutorials/tips_for_best_training_results/.

Rath, S. (2023). YOLOv8: Comprehensive Guide to State of the Art Object Detection. LearnOpenCV. Retrieved 24 March from https://learnopencv.com/ultralytics-YOLOv8/

Ajayi, O. G., Ibrahim, P. O., & Adegboyega, O. S. (2024). Effect of Hyperparameter Tuning on the Performance of YOLOv8 for Multi-Crop Classification on UAV Images. Applied Sciences, 14(13). https://doi.org/10.3390/app14135708.