Resource Management and GA based scheduling for Unmanned-Aerial-Vehicles Communications

1st Mohammed I. Habelalmateen
Department of Computer Technical Engineering
College of Technical Engineering
The Islamic University
Najaf, Iraq
moh.almateen@gmail.com. 



4th Rizgar Rahman Ghafour 
National University of Science and Technology
Dhi Qar, Iraq 
rizgar.r.ghafour@nust.edu.iq
2nd Nejood F. Abdulsattar 
Department of Computer Technical Engineering 
Imam Al-Kadhum College (IKC) 
Al-Diwaniyah, Iraq 
nejodfaisal@gmail.com.




5th Zahraa Hassan Ward
Department of Computer Techniques Engineering 
Mazaya University College
DhiQar, Iraq;
Eng.co.zahraa@mpu.edu.iq. 
3rd Amit Gangopadhyay
Department of Electronics and Communication Engineering 
Erstwhile Sree Vidyanikethan Engineering College 
Mohan Babu University
Tirupati, India;
ag102091@gmail.com. 


6th Fatima Hashim Abbas
Medical Laboratories Techniques Department
Al-Mustaqbal University
Hillah, Iraq
fatimahashim@mustaqbal-college.edu.iq

Abstract— The integration of vehicular communication and unmanned aerial vehicles (UAVs) technology has become a most trending topic and it occupies maximum of the attention of both the industrial and academic sectors. To achieve high quality communication with the ground and the air medium the aerial vehicles are connected to the cellular network so the year energy constraints are normalized. At the time of high speed data transmission the vehicles underwent certain drawbacks like delay during data uplink and high power consumption. To overcome these drawbacks in this article resource management and Generic Algorithm (GA) based scheduling (RMGAS-UAV) is developed for aerial networks based environments. The core modules of RMGAS-UAV are efficient system model and GA based drone scheduling. This models the data transmission quality of the aerial vehicles are highly improved. This network model is designed in the software called NS3 and the parameters which are taken for result calculation are data delivery ratio, network throughput, routing overhead, energy efficiency and energy consumption. From the calculated results it is shown that the RMGAS-UAV obtained better results in terms of energy efficiency and data delivery when compared with the earlier methods.

Keywords— unmanned aerial vehicles (UAVs), Generic Algorithm (GA), RMGAS-UAV, Resource Management and Drone Scheduling.

# Introduction

In order to provide controlled mobility to the high speed vehicles the technology of (unmanned aerial vehicles) UAVs [1-4] are developed which are otherwise called drones [5-8]. This technology gained maximum popularity in the recent decade which is used to provide huge deployment for confidential military and civilian applications [9-11]. Aerial vehicles are used for both vehicular communication and wireless communication [12-16]. The commercial applications provide simple point to point links over certain specific spectrum coverage areas to operate it [17-20]. In recent times cellular based aerial devices or emerged and it provides promising communication to the new aerial users [21-44].

Various applications are continuously developed with the combination of aerial vehicles so that it needs maximum control and communication quality [25-29]. The recent issues which are present in the current updated aerial vehicles are QoS constraints [30-34], energy efficient communication [35-38] during the process of uplink data loading and throughput maximization [39-40]. In order to overcome such drawbacks in this article resource management is concentrated which includes optimization based scheduling to obtain efficient communication in the aerial vehicles. The major contribution of the research is described as follows.

This proposed model is built primarily to provide efficient scheduling and resource management, as well as to obtain correct uplink data transmission of the earlier cars. Drone scheduling based on genetic algorithms and effective system design are the main functions of this model. By using this procedure, the data uplink quality improved, which raises the bar for data transfer.

# Related Works

In [41], developed an reinforcement learning-based spatiotemporal scheduling framework for autonomous UAVs. Additionally, Q-Learning is employed for constraints like limited battery capacity, delays. In [42], proposes a congestion control model for VANETs, employing a combination of K-means and Scatter Search (SS) techniques. K-means clusters messages based on attributes, and SS determines feasible pathways between nodes. In [43] introduced a wireless communication system for multiple UAVs that is compatible with 6G technology. The objective is to maximize the least average attainable rate across users by optimizing the trajectory and resource scheduling of several UAVs.

In [44], introduced a new framework called Reliability Aware Multi-Objective Optimization-Based Routing for VANETs. The system incorporates routing methods and optimization techniques via the use of Enhanced Gaussian Mutation Harmony Searching. In [45], investigated energy-efficient communication methods for UAVs. The challenge entails the simultaneous optimization of trajectory, transmit power, speed, and UAV-to-user scheduling in order to optimize both throughput and energy efficiency. In [46], highlights security challenges in VANETs, particularly the Sybil attack. VANETs, lacking infrastructure, face risks as malicious entities can assume multiple identities, transmitting false messages and compromising road safety. In [47], proposed an investigation into the joint influence of uncertainties in jammer positions and constraints on UAV turning angles in a UAV-enabled WCS. In addition, a powerful alternating algorithm utilizes the block coordinate descent technique, sequential convex approximation, and S-procedure to get a suboptimal solution.

In [48] joint optimization based path planning models are presented mainly to reduce the energy cost of the vehicles which are present at the network. In [49] sleep scheduling algorithm is concentrated to improve the efficiency of the aerial vehicles and as well improved data collection model is incorporated with it mainly to improve the lifetime of the network. In [50] bandwidth optimization with an efficient data collection model is developed mainly to provide large coverage and rapid deployment for the aerial vehicles. All the earlier methods which are described in this section are summarized in table I.

Earlier Research Summary.

| No | Methodology Details | Advantages | Disadvantages |
| --- | --- | --- | --- |
| [41] | Hybrid Offline-Online Optimization | flexible and adaptable | Computation delay is high |
| [45] | UAV-to-User Channel Model | Approach aims to maximize | Overall performance is moderate |
| [46] | Enhanced Gaussian Mutation Harmony Searching | objective decomposition | Power utilization and delay is high |
| [47] | Unmanned Aerial Vehicles (UAVs) | Performance improvement compared to benchmark methods. | High power utilization |
| [48] | VANET technology | Utilization of wireless communication | Data transmission accuracy is moderate |
| [49] | Successive Convex Approximation (SCA) approach | Minimize the total system energy cost | Network latency is high |
| [50] | UAV-Assisted Sleep Scheduling | Collecting nodes residual energy | Produce more power consumption |

# Proposed RMGAS-UAV Approach.

The primary goal of this proposed approach is to optimize and distribute resources across the network's aerial vehicles in an efficient manner. Drone scheduling is provided using genetic algorithms, enabling the network to achieve its highest uplink quality and throughput. “Fig.1”, depicts the RMGAS-UAV's workflow.

RMGAS-UAV Workflow.

## System model.

Let's assume there is an area of interest (AoI) where a total of Ωall SNs are spread in a random and uniform manner. Each SN i in the AoI has a fixed location denoted as Li=[Lxi,Lyi]ϵR2×1 ). The data has been placed in the UAV. The data queue length of SN i in time slot t, denoted as Bi(t),   is affected by both the data sampling rate (s) and the data uploading throughput with the UAV  (i.e Thri(t)), “(1)”.

Bit=min⁡{Bit-1+s-Thrit, B}	

It is important to mention that if the queue length Bit exceeds the storage capacity B, all data sampled after that point would be discarded due to the storage being exhausted. Moreover, it is presumed that the data queue of SN i may be estimated by the UAV by using the predetermined sampling rate s and the historical communication records with each SN. In order to streamline analysis, the activities of the UAV during the whole data collection process are shown using a temporal segmentation approach. Therefore, the trajectory of the UAV may be represented as a series of waypoint coordinates. {Lut}t=1T of which Lu(t)=[Luxt,Luyt] refers the way-point position of the UAV, received by the attached GPS, projected on the ground at the t-th time slot, and the flight length T of the UAV in a data collecting job must satisfy as in “(2)”:

T≤Te	2

Where Te denotes the maximum duration of the UAV's flight. Meanwhile, the trajectory of the UAV from time slot t-2 to time slot t is calculated by “(3)”.

dt=Lut-Lut-1≤Vmax×τi	3

where Vmax represents the highest velocity of the UAV, whereas τ   denotes the time duration between adjacent time slots. Let us assume that the UAV is flying at a constant altitude, denoted as H. The distance between the UAV and Signal Node (SN) i at time slot t is quantified as du,it=Li -Lut. The communication rate of SN i in time slot t may be denoted as “(4)”.

rit=Wlog2(1+αipσi2(H2+du,i2t))	4

where αi represents the power increase of the reference channel in real-time, when the distance du,i=1m and p represents the transmission power, which is initially set to the same value. Moreover, due to the close proximity of the SNs involved in the UAV interaction and the short duration of τ in equation (3), it is assumed that the channel status (i.e. σi2) between each UAV-SN communication link is uniformly distributed and stays unchanged during a time slot. Therefore, it is also assumed that any SN i may establish a reliable communication connection with the UAV at any given moment.

du,i(t)≤dc	5

The quantity dc  reflects the average distance of successful communication between the SN and the UAV when projected onto a two-dimensional plane. OFDMA technology enables the UAV to efficiently allocate communication resources by dividing them into n equally sized and orthogonal resource blocks.  Each communication SN is allotted a bandwidth of W, as shown in “(4)”. The UAV, using OFDMA technology, has the capability to simultaneously facilitate data transfers with a maximum of n adjacent SNs that satisfy the criteria specified in “(5)” during each time slot t, “(6)”.

Ω0(t)≤n, ∀t	6

where Ω0t  represents the collection of selected SNs that the UAV chooses to establish communication with during time slot t. The throughput of SN i in time slot t is determined by “(7)”.

Thrit=minritτc, Bit, ∀i∈Ω0t	7

Where τc represents the length of a data transmission operation inside a single time slot, with the assumption that  τc≪τ . The proposed system model is elaborated in the pseudo code below.

The Pseudo Code of the system model Inputs: UAV network Creation Step 1 - Queue length calculation - Bit=min⁡{Bit-1+s-Thrit, B} Step 2 - Flight length calculation - T≤Te Step 3 – Distance calculation - dt=Lut-Lut-1≤Vmax×τi Step 4 - Communication rate - rit=Wlog2(1+αipσi2(H2+du,i2t)) Step 5 - Communication link - du,i(t)≤dc Step 6 – Adjacent SN Finding - Ω0(t)≤n, ∀t Step 7 - Throughput of SN - Thrit=minritτc, Bit, ∀i∈Ω0t Step 8 – Performance Analysis

## GA based scheduling Process.

In order to overcome the limitations of the greedy-based strategy, our objective is to devise a more sophisticated heuristic technique by using the genetic algorithm. The evolutionary algorithm is being considered due to its high efficiency in finding a suboptimal solution for the disinfection issue, while still meeting tolerable constraints.  It has been previously used in many investigations, which have shown considerable enhancements in performance. The evolutionary algorithm determines the optimal path by considering the fitness of both its ancestors and itself. The inputs of the algorithm are the x and y coordinates of each place, xc and yc, as inputs and produces the optimal route as its output.

Initially, the variables population size Sp, generations Gen, elite size Se, and mutation rate μr are initialized. Next, a loclist is generated that includes the xc and yc coordinates of each place, with each location being referred to as a gene. After initializing all the variables, the genetic algorithm is executed starting from line 19. Initially, a population is generated from a loclist of size Sp  using the createroute function. This function creates a route that is tailored to our specific needs.  A variable v is used to monitor the cumulative energy, which is determined by adding the current et to the total energy of the new location ETloc. After the drone depletes its maximum energy capacity Emax , it will return to the base station (bs) and begin a new route by include the bs in its path. The population comprises a collection of Sp randomly generated paths, referred to as chromosomes,, which may be regarded as prospective candidates for the computation. Once the population is started, a sequence of subsequent processes is executed for G generations to determine the ultimate population. Initially, the fitness function, Φ, is computed using “(8)”:

Φi,j=1ED(i,j)	8

To minimize ED, the reversal of Φ leads to a direct proportionality between a route's performance and its fitness. In this case, the fitness function was calculated for all routes inside a particular scenario as a whole, allowing for the evaluation of the overall effectiveness of the whole scheme rather than individual routes individually. Once Φ has been computed, the chromosomes are arranged in order based on their fitness value. The first selection phase is selecting the top 4 chromosomes from the ranked population to create the mating pool, referred to as Mp. The selection of  Se may be determined as a fraction of the whole population. Afterwards, the remaining chromosomes are assigned a weight and are subsequently selected randomly from the mating pool depending on these weights. The mating pool consists of a group of very skilled progenitors that are responsible for producing a chromosome that is more efficient in terms of energy consumption. The act of selecting parents based on their Φ and  Se is known as elitism. Subsequently, the parents selected from the pool of potential mates are used to produce novel chromosomes, or progeny, by the implementation of the crossover operation, also known as the breeding technique.

The crossover method entails choosing partial routes from both parents in the Mp set and generating a new set of chromosomes that include the offspring's genes. The mutation of the recently established population is a pivotal and last stage. During the process of mutation, there are random swaps of locations or genes within a chromosome. This is of utmost importance as it ensures the presence of a diverse population by preventing local convergence. The technique is repeated for G generations, culminating in a final modified population acquired from the last generation. The final population is once again sorted according on Φ. The first element in the ranking list is chosen as the optimum solution for the issue. Given the scalability and capacity of the genetic algorithm to provide a global solution, it might be seen as the ultimate. A very efficient scheduling algorithm for the disinfection drones in our advanced UV-CDS model. In the following section, we conduct a comprehensive evaluation of the potential algorithms to gauge their performance in relation to time and space complexity.

# Simulation Results.

With the use of the NS3 simulator, we were able to evaluate the performance of the JOSP-UAV [48], SEDC-UAV [49], and JSB-UAV [50] methods in order to conduct an analysis of the proposed RMGAS-UAV. The inputs used in simulation are given in table II.

Input Parameters Table

| Input Parameters | Values |
| --- | --- |
| Running Time | 150 ms |
| Coverage Distance | 2000m*2000m |
| Number of Vehicles | 300 Nodes |
| Vehicles Radius | 150m |
| Vehicles Speed | 30Km/Hr |
| Initial Energy | 100 Joules |
| Transmission Power | 0.500 Joules |
| Receiving Power | 0.050 Joules |

## Data Delivery Ratio.

PDR is a metric that quantifies the proportion of data packets that are successfully delivered compared to the total number of packets sent.

Data Delivery Ratio Calculation.

The “Fig.2”,  shows the Data Delivery Ratio Calculation of RMGAS-UAV and it is resulted with the other models like JOSP-UAV, SEDC-UAV, and JSB-UAV.

The proposed RMGAS-UAV prototype has a remarkable Packet Delivery Ratio of 92%, which showing the capacity to consistently transmit data without errors. This has superior performance compared to JOSP-UAV of 73%, SEDC-UAV of 79%, and JSB-UAV of 85%.

## Throughput.

Throughput quantifies the speed at which data is effectively transferred over the network. The “Fig.3”, shows the Throughput Calculation of RMGAS-UAV and it is resulted with the other models like JOSP-UAV, SEDC-UAV, and JSB-UAV.

Throughput Calculation.

The RMGAS-UAV prototype demonstrates the greatest data transfer rate at 624 kbps, showing its superior efficiency in delivering information at a faster pace. RMGAS-UAV demonstrates superior performance compared to JOSP-UAV at 256 kbps, SEDC-UAV at 354 kbps, and JSB-UAV at 417 kbps,.

## Routing Overhead.

Routing Overhead refers to the supplementary packets used for the management and maintenance of the routing process.

Routing overhead calculation.

The “Fig.4”, shows the Routing Overhead Calculation of RMGAS-UAV and it is resulted with the other models like JOSP-UAV, SEDC-UAV, and JSB-UAV.

The proposed RMGAS-UAV, as suggested, exhibits the lowest routing overhead with a total of 324 packets which surpasses the existing techniques JOSP-UAV of 654 packets, SEDC-UAV of 574 packets, and JSB-UAV of 521 packets, showing efficient use of resources in network routing.

## Energy Efficiency.

Energy efficiency is a metric that quantifies the system's ability to optimally use energy resources for communication purposes. The “Fig.5”, shows the Energy Efficiency Calculation of RMGAS-UAV and it is resulted with the other models like JOSP-UAV, SEDC-UAV, and JSB-UAV.

Energy Efficiency Calculation.

The RMGAS-UAV prototype demonstrates superior energy efficiency, achieving energy usage of 524 Joules for data transmission. This demonstrates superior performance compared to the existing technique JOSP-UAV (256 Joules), SEDC-UAV (296 Joules), and JSB-UAV (345 Joules), indicating that it can transport data more effectively while using less energy.

## Energy Consumption.

Energy Consumption refers to the aggregate quantity of energy used for communication. The “Fig.6”, shows the energy Consumption Calculation of RMGAS-UAV and it is resulted with the other models like JOSP-UAV, SEDC-UAV, and JSB-UAV.

Energy Consumption Calculation.

The RMGAS-UAV exhibits the minimum energy consumption, namely at 186 Joules, revealing its need for the least amount of energy for data transmission. On the other hand, JOSP-UAV, SEDC-UAV, and JSB-UAV have greater energy consumption levels, with JOSP-UAV using 356 Joules, SEDC-UAV consuming 324 Joules, and JSB-UAV consuming 276 Joules.

# Conclusion.

In this paper, the efficiency of the data uplink quality and the throughput maximization are concentrated. To improve the quality of service requirements of the aerial vehicles in the article effective resource management-based optimization is concentrated. As a result, the proposed model obtained maximum efficiency and delivery ratio at the time of data up link transmission and as well it reaches maximum throughput when compared to the baseline schemes.

##### References

R. Q. Malik, K. N. Ramli, Z. H. Kareem, M. I. Habelalmatee, A. H. Abbas, and A. Alamoody, "An Overview on V2P Communication System: Architecture and Application," in 2020 3rd International Conference on Engineering Technology and its Applications (IICETA), pp. 174-178, 2020. doi: https://doi.org/10.1109/IICETA50496.2020.9318863.

Y.S. Jghef, M.J.M. Jasim, H.M.A. Ghanimi, A.D. Algarni, N.F. Soliman, W. El-Shafai, S.R.M. Zeebaree, A. Alkhayyat, A.S. Abosinnee, N.F. Abdulsattar, A.H. Abbas, H.M. Hariz, and F.H. Abbas, "Bio-Inspired Dynamic Trust and Congestion-Aware Zone-Based Secured Internet of Drone Things (SIoDT)," Drones, vol. 6, no. 11, pp. 337, 2022. https://doi.org/10.3390/drones6110337.

G.G. Shayea, D.A. Mohammed, A.H. Abbas, and N.F. Abdulsattar, "Privacy-Aware Secure Routing through Elliptical Curve Cryptography with Optimal RSU Distribution in VANETs," Designs, vol. 6, no. 6, pp. 121, 2022. doi: https://doi.org/10.3390/designs6060121.

M.H. Hassan, A.H. Abbas, M.H. Mutar, M.A. Jubair, S.A. Rashid, A.J. Ahmed, M.I. Habelalmateen, A. Mohammed, and L. Audah, "Investigative Study on Routing Protocols in Flying Ad Hoc Networks Environment," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 460-463, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888600.

N.F. Abdulsattar, F. Abedi, H.M.A. Ghanimi, S. Kumar, A.H. Abbas, A.S. Abosinnee, A. Alkhayyat, M.H. Hassan, and F.H. Abbas, "Botnet Detection Employing a Dilated Convolutional Autoencoder Classifier with the Aid of Hybrid Shark and Bear Smell Optimization Algorithm-Based Feature Selection in FANETs," Big Data and Cognitive Computing, vol. 6, no. 4, article no. 112, 2022. DOI: https://doi.org/10.3390/bdcc6040112.

V. O. Nyangaresi, I. M. Al-Joboury, K. A. Al-sharhanee, A. H. Najim, A. H. Abbas, and H. M. Hariz, "A biometric and physically unclonable function–Based authentication protocol for payload exchanges in internet of drones," e-Prime - Advances in Electrical Engineering, Electronics and Energy, vol. 7, pp. 100471, 2024. DOI: https://doi.org/10.1016/j.prime.2024.100471.

N.F. Abdulsattar, A.H. Abbas, M.H. Mutar, M.H. Hassan, M.A. Jubair, and M.I. Habelalmateen, "An Investigation Study for Technologies, Challenges and Practices of IoT in Smart Cities," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 554-557, 2022. DOI: https://doi.org/10.1109/IICETA54559.2022.9888474.

A. H. Abbas, N. F. Abdulsattar, H. S. Mansour, M. H. Mutar, M. I. Habelalmateen, L. Audah, N. A. M. Alduais, and A. Mohammed, "A New Hybrid Approach Cluster-Heads Election to reduce the number of clusters in VANETs," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 436-440, 2022. DOI: https://doi.org/10.1109/IICETA54559.2022.9888442.

M. I. Habelalmateen, A. H. Abbas, L. Audah, and N. A. M. Alduais, "Dynamic multiagent method to avoid duplicated information at intersections in VANETs," Telkomnika (Telecommunication Computing Electronics and Control), vol. 18, no. 2, pp. 613-621, 2020. DOI: http://doi.org/10.12928/telkomnika.v18i2.13947.

M. K. Hasan, M. M. Ahmed, N. F. Wani, A. H. Abbas, L. M. Alkwai, S. Islam, A. K. M. A. Habib, and R. Hassan, "Dynamic load modeling for bulk load-using synchrophasors with wide area measurement system for smart grid real-time load monitoring and optimization," Sustainable Energy Technologies and Assessments, vol. 57, p. 103190, 2023. doi: https://doi.org/10.1016/j.seta.2023.103190.

A. H. Abbas, A. J. Ahmed, and S. A. Rashid, "A Cross-Layer Approach MAC/NET with Updated-GA (MNUG-CLA)-Based Routing Protocol for VANET Network," World Electric Vehicle Journal, vol. 13, no. 5, article no. 87, 2022. DOI: https://doi.org/10.3390/wevj13050087.

H. S. Mansour, M. H. Mutar, I. A. Aziz, S. A. Mostafa, H. Mahdin, A. H. Abbas, M. H. Hassan, N. F. Abdulsattar, and M. A. Jubair, "Cross-Layer and Energy-Aware AODV Routing Protocol for Flying Ad-Hoc Networks," Sustainability, vol. 14, no. 15, article no. 8980, 2022. DOI: https://doi.org/10.3390/su14158980.

M. A. Jubair, S. A. Mostafa, D. A. Zebari, H. M. Hariz, N. F. Abdulsattar, M. H. Hassan, A. H. Abbas, F. H. Abbas, A. Alasiry, and M. Turki-Hadj, "A QoS Aware Cluster Head Selection and Hybrid Cryptography Routing Protocol for Enhancing Efficiency and Security of VANETs," IEEE Access, vol. 10, pp. 124792-124804, 2022. doi: https://doi.org/10.1109/ACCESS.2022.3224466.

M. I. Habelalmateen, A. J. Ahmed, A. H. Abbas, and S. A. Rashid, "TACRP: Traffic-Aware Clustering-Based Routing Protocol for Vehicular Ad-Hoc Networks," Designs, vol. 6, no. 5, article no. 89, 2022. DOI: https://doi.org/10.3390/designs6050089.

S. A. Rashid, M. I. Habelalmateen, A. J. Ahmed, A. H. Abbas, M. A. Jubair, M. H. Hassan, A. Abdulhadi, S. Hammed, and L. Audah, "Congestion Aware Genetic Q-learning Based RPL Routing Protocol for Vehicle Ad-Hoc Networks," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 464-469, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888583.

A. H. Abbas, A. J. Ahmed, S. A. Rashid, N. F. Abdulsattar, M. H. Hassan, M. A. Jubair, and M. I. Habelalmateen, "Predictive Hybrid Routing with Multi Objective Optimization Model for Opportunistic Routing for VANET," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 470-476, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888274.

A. H. Najim, A. H. Abbas, K. Al-sharhanee, and H. M. Hariz, "Reinforcement Learning-based Topology-Aware Routing Protocol with Priority Scheduling for Internet of Drones in Agriculture Application," International Journal of Intelligent Engineering and Systems, vol. 16, no. 5, pp. 395-405, 2023. doi: https://doi.org/10.22266/ijies2023.1031.34.

A. H. Abbas, L. Audah, and N. A. M. Alduais, "An Efficient Load Balance Algorithm for Vehicular Ad-Hoc Network," in 2018 Electrical Power, Electronics, Communications, Controls and Informatics Seminar (EECCIS), pp. 207-212, 2018. https://doi.org/10.1109/EECCIS.2018.8692827.

A. H. Alwan, A. A. Aziz, M. A. Jubair, A. H. Abbas, M. H. Hassan, S. Alheejawi, N. F. Abdulsattar, A. S. Mustafa, and M. I. Habelalmateen, "Monitoring the Impact of the Nodes Density on the MANETs’ Environment," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 546-549, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888339.

Mostafa, S.A., Mustapha, A., Ramli, A.A., Jubair, M.A., Hassan, M.H., Abbas, A.H. (2021). Comparative Analysis to the Performance of Three Mobile Ad-Hoc Network Routing Protocols in Time-Critical Events of Search and Rescue Missions. In: Cassenti, D., Scataglini, S., Rajulu, S., Wright, J. (eds) Advances in Simulation and Digital Human Modeling. AHFE 2020. Advances in Intelligent Systems and Computing, vol 1206. Springer, Cham. https://doi.org/10.1007/978-3-030-51064-0_16.

M. H. Hassan, M. A. Jubair, A. H. Abbas, A. J. Ahmed, S. A. Rashid, M. H. Mutar, H. S. Mansour, A. Ali, and M. I. Habelalmateen, "AODV based Crow Search Algorithm for Improving the QoS of MANET," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 424-428, 2022. DOI: https://doi.org/10.1109/IICETA54559.2022.9888484.

M. A. Jubair, M. H. Hassan, S. A. Mostafa, H. Mahdin, A. Mustapha, L. H. Audah, F. S. Shaqwi, and A. H. Abbas, "Competitive analysis of single and multi-path routing protocols in MANET," Indonesian Journal of Electrical Engineering and Computer Science, vol. 19, no. 1, pp. 293-300, 2020. http://doi.org/10.11591/ijeecs.v19.i1.pp293-300.

S. A. Rashid, A. J. Ahmed, M. H. Hassan, A. H. Abbas, M. I. Habelalmateen, M. A. Jubair, L. Audah, and W. Al-Azzawi, "Ranking Based Routing Protocol for Residual Energy Estimation in Wireless Sensor Network Based IoT Network," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 453-459, 2022. https://doi.org/10.1109/IICETA54559.2022.9888349.

A. J. Ahmed, A. H. Abbas, S. A. Rashid, M. A. Jubair, M. H. Hassan, A. Abdulhadi, N. F. Abdulsattar, and M. I. Habelalmateen, "Congestion Aware Q-Learning (CAQL) in RPL Protocol – WSN based IoT Networks," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 429-435, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888322.

N. F. Abdulsattar, D. A. Mohammed, A. Alkhayyat, S. Z. Hamed, H. M. Hariz, A. S. Abosinnee, A. H. Abbas, M. H. Hassan, M. A. Jubair, F. H. Abbas, A. D. Algarni, N. F. Soliman, and W. El-Shafai, "Privacy-Preserving Mobility Model and Optimization-Based Advanced Cluster Head Selection (P2O-ACH) for Vehicular Ad Hoc Networks," Electronics, vol. 11, no. 24, p. 4163, 2022, DOI: https://doi.org/10.3390/electronics11244163.

F. Abedi, S. R. M. Zeebaree, Z. S. Ageed, H. M. A. Ghanimi, A. Alkhayyat, M. A. M. Sadeeq, S. N. Mahmood, A. S. Abosinnee, Z. H. Kareem, A. H. Abbas, W. K. Al-Azzawi, M. M. Jaber, and M. Dauwed, "Severity Based Light-Weight Encryption Model for Secure Medical Information System," Computers, Materials & Continua, vol. 74, no. 3, pp. 5691-5704, 2023. DOI: https://doi.org/10.32604/cmc.2023.034435.

A. H. Kadhim, A. M. Razaq, M. A. Jubair, N. F. Abdulsattar, A. H. Abbas, M. H. Hassan, A. Jamal Ahmed, S. AbdulJabbar Rashid, and M. I. Habelalmateen, "A Real-Time Low-Cost Remote Doctor Monitoring System," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 558-561, 2022. doi: https://doi.org/10.1109/IICETA54559.2022.9888536.

F. Abedi, H. M. A. Ghanimi, A. D. Algarni, N. F. Soliman, W. El-Shafai, A. H. Abbas, Z. H. Kareem, H. M. Hariz, and A. Alkhayyat, "Computational Intelligence Driven Secure Unmanned Aerial Vehicle Image Classification in Smart City Environment," Computer Systems Science and Engineering, vol. 47, no. 3, pp. 3127-3144, 2023. DOI: https://doi.org/10.32604/csse.2023.038959.

I. Ahmad, S. Hussain, S. N. Mahmood, H. Mostafa, A. Alkhayyat, M. Marey, A. H. Abbas, and Z. Abdulateef Rashed, "Co-Channel Interference Management for Heterogeneous Networks Using Deep Learning Approach," Information, vol. 14, no. 2, article no. 139, 2023. DOI: https://doi.org/10.3390/info14020139.

A. M. Abdulkarem, F. Abedi, H. M. A. Ghanimi, S. Kumar, W. K. Al-Azzawi, A. H. Abbas, A. S. Abosinnee, I. M. Almaameri, and A. Alkhayyat, "Robust Automatic Modulation Classification Using Convolutional Deep Neural Network Based on Scalogram Information," Computers, vol. 11, no. 11, article no. 162, 2022. DOI: https://doi.org/10.3390/computers11110162.

S. Alsubai, A. K. Dutta, A. H. Alkhayyat, M. M. Jaber, A. H. Abbas, and A. Kumar, "Hybrid deep learning with improved Salp swarm optimization based multi-class grape disease classification model," Computers and Electrical Engineering, vol. 108, p. 108733, 2023. DOI: https://doi.org/10.1016/j.compeleceng.2023.108733.

I. S. Al-Hawary, E. Ali, S. M. H. Kamona, L. Hussain Saleh, A. S. Abdulwahid, D. N. Al-Saidi, M. S. Alhassan, F. A. Rasen, H. Abdullah Abbas, A. Alawadi, A. H. Abbas, and M. Sina, "Prediction of geomechanical bearing capacity using autoregressive deep neural network in carbon capture and storage systems," Heliyon, vol. 9, no. 11, pp. e21913, Nov. 2023. doi: https://doi.org/10.1016/j.heliyon.2023.e21913.

F. Abedi, H. M. A. Ghanimi, M. A. M. Sadeeq, A. Alkhayyat, Z. H. Kareem, S. N. Mahmood, A. H. Abbas, A. S. Abosinnee, W. K. Al-Azzawi, M. M. Jaber, and M. Dauwed, "Hybrid Deep Learning Enabled Load Prediction for Energy Storage Systems," Computers, Materials & Continua, vol. 75, no. 2, pp. 3359-3374, 2023. DOI: https://doi.org/10.32604/cmc.2023.034221.

M. R. M. Alsemawi, M. H. Mutar, E. H. Ahmed, H. O. Hanoosh, and A. H. Abbas, "Emotions recognition from human facial images based on fast learning network," Indonesian Journal of Electrical Engineering and Computer Science, vol. 30, no. 3, pp. 1478-1487, 2023. doi: http://doi.org/10.11591/ijeecs.v30.i3.pp1478-1487.

J. Tang, A. K. Abbas, N. A. Koka, N. Sadoon, J. K. Abbas, R. A. Abdalhuseen, M. Abosaooda, N. M. Ahmed, and A. H. Abbas, "Optimization of thermal biofuel production from biomass using CaO-based catalyst through different algorithm-based machine learning approaches," Case Studies in Thermal Engineering, vol. 50, p. 103419, 2023. doi: https://doi.org/10.1016/j.csite.2023.103419.

F. Abedi, H. M. A. Ghanimi, A. D. Algarni, N. F. Soliman, W. El-Shafai, A. H. Abbas, Z. H. Kareem, H. M. Hariz, and A. Alkhayyat, "Chimp Optimization Algorithm Based Feature Selection with Machine Learning for Medical Data Classification," Computer Systems Science and Engineering, vol. 47, no. 3, pp. 2791–2814, 2023. doi: https://doi.org/10.32604/csse.2023.038762.

M. A. A. Albadr et al., "Online sequential extreme learning machine approach for breast cancer diagnosis," Neural Computing and Applications, vol. Volume, no. Issue, pp. Page, 2024. DOI: https://doi.org/10.1007/s00521-024-09617-x.

M. S. Alshuhri, S. G. Al-Musawi, A. A. Al-Alwany, H. Uinarni, I. Rasulova, P. Rodrigues, A. T. Alkhafaji, A. M. Alshanberi, A. H. Alawadi, and A. H. Abbas, "Artificial intelligence in cancer diagnosis: Opportunities and challenges," Pathology - Research and Practice, vol. 253, pp. 154996, 2024. https://doi.org/10.1016/j.prp.2023.154996.

A. Abdulbari, S. K. Abdul Rahim, F. Abedi, P. J. Soh, A. Hashim, R. Qays, S. Ahmad, and M. Y. Zeain, "Single-Layer Planar Monopole Antenna-Based Artificial Magnetic Conductor (AMC)," International Journal of Antennas and Propagation, vol. 2022, pp. 6724175, 2022. https://doi.org/10.1155/2022/6724175.

S. Saleem, A. H. Jabbar, M. H. Jameel, A. Rehman, Z. H. Kareem, A. H. Abbas, Z. Ghaffar, S. A. Razzaq, R. A. Pashameah, E. Alzahrani, E.-P. Ng, and S. M. Sapuan, "Enhancement in structural, morphological, and optical properties of copper oxide for optoelectronic device applications," Nanotechnology Reviews, vol. 11, no. 1, pp. 2827-2838, 2022.DOI: https://doi.org/10.1515/ntrev-2022-0473..

O. Bouhamed, H. Ghazzai, H. Besbes, and Y. Massoud, "A generic spatiotemporal scheduling for autonomous UAVs: A reinforcement learning-based approach," IEEE Open Journal of Vehicular Technology, vol. 1, 2020, pp. 93-106, doi: 10.1109/OJVT.2020.2979559.

A. H. Abbas, H. Mansour, and A. Al-Fatlawi, "Self-Adaptive Efficient Dynamic Multi-Hop Clustering (SA-EDMC) Approach for Improving VANET’s Performance," International Journal of Interactive Mobile Technologies (iJIM), vol. 16, no. 14, pp. 136-151, Jul. 2022. DOI: https://doi.org/10.3991/ijim.v16i14.31081.

J. Wang, Z. Na, and X. Liu, "Collaborative design of multi-UAV trajectory and resource scheduling for 6G-enabled internet of things," IEEE Internet of Things Journal, vol. 8, no. 20, 2020, pp. 15096-15106, doi: 10.1109/JIOT.2020.3031622.

A. H. Abbas, M. I. Habelalmateen, L. Audah, and N. A. M. Alduais, "A novel intelligent cluster-head (ICH) to mitigate the handover problem of clustering in VANETs," International Journal of Advanced Computer Science and Applications, vol. 10, no. 6, pp. 281-287, 2019. DOI: https://doi.org/10.14569/IJACSA.2019.0100627.

S. Ahmed, M. Z. Chowdhury, and Y. M. Jang, "Energy-efficient UAV-to-user scheduling to maximize throughput in wireless networks," IEEE Access, vol. 8, 2020, pp. 21215-21225, doi: 10.1109/ACCESS.2020.2969357.

A. H. Abbas, A. J. Ahmed, S. A. Rashid, M. A. Jubair, N. F. Abdulsattar, H. S. Mansour, and M. I. Habelalmateen, "Hybrid ANT Based Continuous Cuckoo Search Optimization Algorithm for Vehicle Adhoc Networks," in 2022 5th International Conference on Engineering Technology and its Applications (IICETA), pp. 447-452, May 31-June 1, 2022. https://doi.org/10.1109/IICETA54559.2022.9888404.

Y. Gao et al., "Robust trajectory and scheduling design for turning angle-constrained UAV communications," in 2020 International Conference on Wireless Communications and Signal Processing (WCSP), 2020, pp. 962-967, doi: 978-1-7281-7236-1/20.

Doyoung Kim and Seongah Jeong, “Joint Optimization of Offloading Scheduling and Path Planning for Space-air-ground Integrated Edge Computing Systems”, International Conference on Information and Communication Technology Convergence (ICTC), 2022, doi: 10.1109/ICTC55196.2022.9952547.

Maowu Zhou, Hongbin Chen, et.al, “UAV-Assisted Sleep Scheduling Algorithm for Energy-Efficient Data Collection in Agricultural Internet of Things”, IEEE Internet of Things Journal, vol. 9, no. 13, pp. 11043 – 11056, 2022, doi: 10.1109/JIOT.2021.3125971

Yue Fu, Demin Li, et.al, “Joint Speed and Bandwidth Optimized Strategy of UAV-Assisted Data Collection in Post-Disaster Areas”, Mediterranean Communication and Computer Networking Conference (MedComNet), 2022, doi: 10.1109/MedComNet55087.2022.9810444.