BER Analysis of OIRS-Assisted Underwater Optical Wireless Communication Systems

Amel Aissaoui 
Department of electronics 
Constantine 1 University
Constantine, Algeria
aissaoui.amel@umc.edu.dz
0000-0002-8790-0858
Latifa Hacini 
Department of electronics
Constantine 1 University
Constantine, Algeria
hacini.latifa@umc.edu.dz

Ghada Amirech 
Department of electronics
Constantine 1 University
Constantine, Algeria
amirechghada@gmail.com

Ali Lokmane Saadi
Department of electronics
Constantine 1 University
Constantine, Algeria
saadialilokmane@gmail.com

Abstract—Optical intelligent reflecting surfaces (OIRSs) introduce a new perspective in wireless communication systems by enabling dynamic control over the propagation characteristics of optical signals. In underwater optical wireless communication (UWOC) environments, this technology offers an effective approach to mitigating the pronounced degradation of the channel induced by physical obstructions. This study explores the performance enhancement obtained by incorporating OIRSs into UWOC links. By employing the Laplace transform and the Fox’s H-function, closed-form analytical expressions for the probability density (PDF) and cumulative distribution (CDF) functions as well as the average bit error rate (ABER) are derived, while explicitly accounting for the effects of underwater attenuation and turbulence. Numerical evaluations highlight the influence of key system parameters, including communication distance, source depth, type of water, and the OIRS array size. The results confirm that the deployment of OIRSs leads to a substantial improvement in both the robustness and efficiency of UWOC systems.

Keywords—UWOC, OIRS, attenuation, turbulence.

# Introduction

Growing interest has been directed toward the internet of underwater things (IoUT), which aims to provide seamless connectivity beneath the water’s surface. The concept focuses on linking underwater sensor networks (USNs) and autonomous underwater vehicles (AUVs) through dependable wireless communication. Such connectivity is essential for a wide range of underwater operations, including offshore oil and gas exploration, long-term environmental monitoring, underwater navigation, and the preservation of marine ecosystems [1], [2].

IoUT relies on three primary communication media: acoustic, radio frequency (RF), and optical. Acoustic waves enable long-range transmission but at very low data rates, while RF provides higher data rates over limited distances. Optical signals strike a balance, offering high data rates and low latency in medium ranges [3]. They also use the large unlicensed optical spectrum, making underwater optical wireless communication (UOWC) cost-effective and energy-efficient. However, optical links are highly vulnerable to obstacles such as marine life and seabed formations, which may cause complete blockages. To address this, non-line-of-sight (NLoS) configurations based on the principle of total internal reflection (TIR) at the sea surface have been proposed [4]. Nevertheless, varying sea surface slopes make this approach unreliable for consistent signal redirection. Recently, intelligent reflecting surfaces (IRSs) have been proposed to mitigate these challenges by redirecting signals, extending coverage, and avoiding blockages [5]. IRSs are particularly attractive in marine network planning, as they can be deployed on existing infrastructure such as ships, AUVs, or buoys, while requiring less hardware complexity than conventional relay nodes.

Although research on IRS-assisted UOWC remains limited, several studies have highlighted its potential. In [5], the outage probability, ABER, and channel capacity of BPSK-based UOWC were evaluated under turbulence, attenuation, and pointing errors, using a generalized Gamma distribution. Significant performance gains, comparable to atmospheric OWC, were reported. In [6], outage performance was investigated across different water types (characterized by chlorophyll concentration), with pointing errors modeled by a Rayleigh distribution and turbulence by a log-normal distribution under the OTOPS spectrum. Results also showed that the reflection coefficient strongly impacts performance. In [7], mirror-based OIRSs were compared with planar mirror surfaces (PMS) under exponential and generalized Gamma (EGG) channel models, demonstrating clear advantages of IRS deployment. In [8], the outage performance of IRS-assisted UOWC with higher-order mode optical beams was analyzed, where the underwater channel was modeled by the Exponentiated Weibull (EW) distribution and beam misalignment by the Hoyt distribution. Finally, [9] investigated the performance of multi-reflecting element RIS-assisted UWOC systems through outage probability and ergodic capacity metrics. The analysis also considered oceanic gamma-gamma turbulence fading as well as pointing errors induced by beam jitter and RIS jitter.

Based on the earlier context, the key contributions of our work are outlined below:

- We analyze an OIRS-UWOC system and compare its performance with conventional NLoS-UWOC configuration, explicitly considering underwater attenuation and turbulence.

- The heterogeneity of underwater environments and water types is considered to provide a more accurate and realistic performance evaluation.

Comparative Review of This Study with Respect to Recent Related Research.

| Ref. | Turbulence modeling | Turbulence heterogeneity | Pointing error | Attenuation | OIRS-Gain optimization | Performance metric |
| --- | --- | --- | --- | --- | --- | --- |
| [5] | Gamma-Gamma | × |  |  | × | BER, OP, CAP |
| [6] | Log-normal | × |  |  | × | OP |
| [7] | EGG | × | × |  |  | BER, OP, CAP |
| [8] | EW | × |  |  | × | OP |
| [9] | Gamma-Gamma | × |  |  | × | OP |
| Our work | Log-normal |  | × |  |  | BER |

- A sine-cosine algorithm (SCA) is employed to optimize the orientation of IRS elements, maximizing the OIRS gain.

- An approximate composite channel model is developed by integrating propagation loss and log-normal turbulence, which is then used to derive and analyze the ABER performance of OIRS-UWOC. Table 1 highlights a comparative summary of the present study and recent literature.

The paper’s organization is as follows: Sections II and III introduce the OIRS-UWOC system and its channel model. Section IV presents the performance analysis with a particular focus on ABER. Section V reports the numerical evaluation, and Section VI concludes the paper.

# System Concept

As shown in Fig. 1, the UWOC link considered in this work features a blocked direct path between the optical source and the destination due to obstacles or mobile underwater objects. To restore the communication link, an OIRS-assisted connection is employed. The OIRS is composed of an (n×m) array of steerable mirror elements and is assumed to operate with complete channel state information (CSI).

The source is located at a depth ys beneath the air–water interface, while the receiver is positioned at depth yd, and the horizontal distance separating them is dsd. The information signal X is intensity-modulated onto an LED source of power Ps and directed toward the OIRS. After reflection from the OIRS, the optical beam is delivered to the destination, where a focusing lens concentrates the light onto a PIN photodetector that measures the received optical intensity.

OIRS-UWOC system model.

- The corresponding detected signal at the photodetector is formulated as [7]

- Z=ηGPsATX+n		         (1)

- where, η denotes the photodetector responsivity, G the OIRS-UWOC system gain, A accounts for link attenuation, T the effect of underwater turbulence, and n the additive white Gaussian noise (AWGN) with standard deviation (std) σn.

- At the receiver, the instantaneous SNR, ν, is written as

- ν=ηGPsAσn2T2=νT2		         (2)

- with, ν=ηGPsAσn2 is the average electrical SNR.

# Channel Model

## Underwater Attenuation

- The primary factors degrading optical communication in underwater  environments are absorption and  scattering, which together attenuate the optical beam. This attenuation can be modeled as [5]–[11]

- A=e-C(λ)dsr+drd		         (3)

where C(λ) denotes the beam extinction coefficient. At a wavelength of 520 nm, its values are 0.0511 m−1, 0.0588 m−1, 0.2922 m−1, associated with pure seawater, clear ocean water, and coastal water, respectively [10]. dsr and drd denote the ranges from the source to the OIRS and from the OIRS to the receiver, respectively..

## Underwater Turbulence

Underwater turbulence, caused by temperature and salinity variations, induces fluctuations in the received optical power. Its strength is characterized by refractive index variations in the aquatic medium. In this work, a log-normal distribution is employed to characterize turbulence, whose PDF is given by [11], [13]

fT(T)=12σT(2π)Te-(ln(T)-2mT)28σT2	         (4)

Here, σT and mT denote the std and mean of the logarithmic amplitude coefficient, respectively.

To account for turbulence heterogeneity with depth, we employ the layered UWOC model in [11], where the source–OIRS (or receiver–OIRS) path is divided into K layers [11]

T=j=1KTj		         (5)

with Tj the turbulence coefficient of the jth layer. If Tj∼LNmTj,σtj2, then T∼LN∑mTj,∑σtj2 [11].

To facilitate mathematical analysis and closed-form derivations, the log-normal PDF is substituted with the hyperbolic tangent-based log-normal (HTLN) model, which is characterized by the scale and shape parameters a and b as follows [12]

fT(T)=be2aTb-1(1+e2aTb)2		         (6)

## OIRS Gain

The OIRS redirects the optical signal to the receiver by adjusting the angles of yaw (β) and roll (Γ) of its mirror elements (see Fig. 2), without altering the signal phase or amplitude [14]. The gain of an OIRS element (k, l) is given by [7], [14]

Gk,lβ,Γ=ερϱ2(χ+1)APD2πdsr2drd2 sin2(ΨFOV)cosχ(Φk,ls)cos(Ψk,ls)       ×cosΦk,ldcosΨk,ldTΨQΨ;                                   0≤Ψdk,l⩽ΨFOV0;                              otherwise                (7)

here, ε denotes the OIRS reflection coefficient, ρ stands for the LED conversion efficiency, ϱ is the refractive index, and APD is the photodiode area. χ represents the Lambertian order, T(Ψ) corresponds to the gain of the optical filter, while ΨFOV indicates the receiver field of view. The angles Φk,ls and Ψk,ls describe, respectively, the irradiance and incidence directions between the source and the k, lth OIRS element, while Φk,ld and Ψk,ld denote the corresponding angles from the OIRS element toward the receiver [7], [14]. Finally, the cosine of the irradiance angle is computed following the formulations presented in [7], [14].

cosΦk,ld=(xk,l-xd)drdsinβcosΓ+yk,l-yddrd          ×cos⁡(β)cos⁡(Γ)+(zk,l-zd)drdsin⁡(Γ);-π2≤β≤+π2and-π2≤Γ≤+π2    (8)

(a)                                                 (b)

Orientation angles of an OIRS element: (a) yaw angle, (b) roll angle.

- Here, xk,l, yk,l, zk,ldenotes the coordinates of the k,lth OIRS element, while xd, yd, zd specifies the receiver’s location. Consequently, the overall channel gain provided by an OIRS composed of n×m elements can be expressed as

- G=k=1nl=1mGk,l(β,Γ) 		         (9)

Maximizing G requires optimizing 2nm orientation variables (β,Γ). To further reduce computational complexity, the Sine–Cosine Algorithm (SCA) is adopted as a population-based stochastic optimization framework, distinguished by its low implementation cost, rapid convergence characteristics, and robustness against convergence to suboptimal solutions [14]. The optimization process, carried out by a centralized controller interfaced with the OIRS elements, is detailed in Algorithm 1.

Algorithm 1: SCA-Based Optimization of the OIRS Gain

Input : - Number of agents N - Maximum iteration T Output : - Optimal orientation angles β*, Γ* Initialization : - Randomly initialize N agents βi, Γi - Evaluate the link gain Gβ,Γ using (7) - Set the best agent as the initial destination point H0 Iterations : (for t=1 to T) - For each agent i: - Update position using the sine-cosine rule: β,Гt+1=β,Гt+r1sinr2 r3 Ηt-β,Гt ; r4<0.5 β,Гt+r1cosr2 r3 Ηt-β,Гt; r4≥0.5 - Update r1=a-taT - Randomly generate r2∈[0, 2π], r3∈[0, 2], r4∈[0, 1] - Enforce bounds: -π2≤β≤+π2and-π2≤Γ≤+π2 - Evaluate updated gain Gβ,Γ - Update destination point Ht as the agent with maximum gain.

# ABER Analysis

The OIRS-UWOC system performance is assessed through the ABER, which reflects the probability of incorrect bit reception and serves as a standard measure of link quality. The generalized expression for ABER is given in [7], [15], [16] as

ABER=δ2Γ(p)i=1Lqip0+∞νp-1e-qiνFν(ν)dν	       (10)

where the parameter set δ, p, qi, L depends on the selected modulation and detection scheme [15], [17]. Let Fν(ν) denote the cumulative distribution function (CDF) of ν. It can be formulated as

Fν(ν)=0νfν(x)dx		       (11)

where, fν(ν) represents the PDF of ν. Based on (2) and (6), and after several algebraic manipulations, fν(ν) is derived as

fν(ν)=be2aνννb2-11+e2aννb22	       (12)

Applying (1.126) from [18], (12) is reformulated using the Fox H-function as follows

fν(ν)=be2aνννb2-1H1,11,1e2aννb2(-1,1)(0,1)	       (13)

Substituting (13) into (11) and using (2.53) from [18], the obtained CDF is given by

Fν(ν)=be2aννb2H2,21,2e2aννb21-b2,b2 (-1,1)0,1-b2,b2     (14)

Finally, inserting (14) into (10) and applying the Laplace transform, the ABER is expressed as

ABER=δ b e2a νp2 Γ(p)i=1L(νqi)-b2-p

×H3,21,3e2a(νqi)b21-b2,b2 (-1,1)1-p-b2,b2 0,1-b2,b2(15)

# Findings evaluation

The ABER performance of the OIRS-assisted UWOC system is analyzed according to underwater parameters. Unless otherwise specified in the figures, system parameters are set according to Table II. The evaluation is carried out through MATLAB-based Monte Carlo simulations with 107 realizations.

Fig. 3 highlights the influence of water type on the performance of the OIRS-UWOC system. As the attenuation coefficient C(λ) increases, greater optical transmit power is essential for compensate for the intensified absorption and scattering effects within the medium. To maintain an ABER of 10−3, the minimum required transmit power is approximately 17.8 dBm for pure seawater, 42 dBm for clear ocean water, and 60 dBm for coastal water.

Setup Parameters for Numerical Evaluation

| Parameter | Symbol | Value |
| --- | --- | --- |
| Number of OIRS elements | (n×m) | (20×20) |
| Photodetector responsivity | η | 0.8 A/W |
| Transmitter and receiver depths | (ys, yd) | (25 m, 25 m) |
| Photodiode area | APD | 0.0314 m2 |
| Optical filter gain | TΨ | 1 |
| Receiver field of view | ΨFOV | 85° |
| Modulation parameters | δ, p, qi, L | 1, 12, 12i, 1 |
| Lambertian order | χ | 2 |
| OIRS reflection coefficient | ε | 0.8 |
| Total node separation | dsd | 100 m |
| AWGN variance | σn2 | 10-14 A2/GHz |
| HTLN parameters | {(a,b)}K=14 | (0.18, 3) |
|  |  | (0.39, 2) |
|  |  | (0.51, 2) |
|  |  | (0.63, 2) |

Variation of ABER across various aquatic environments.

Fig. 4 displays the ABER variation as a function of transmitter depth (ys= 5, 25, 55, 85, 125 m). The receiver is located 25 m below the OIRS plane, with each water layer thickness set to 30m. When the transmitter is positioned very close to the OIRS (ys= 5 m), the incident light strikes the OIRS surface at relatively large angles, reducing the efficiency of reflection toward the receiver. For ys > 5 m, ABER increases with depth as a result of higher signal attenuation caused by the longer propagation path between transmitter and receiver.

Fig. 5 depicts the ABER variation with transmitted power or different transmitter-receiver distances (dsd). As dsdincreases, the overall path length (dsr+drd) becomes longer, which effectively places the receiver farther from the OIRS and consequently degrades the OIRS-UWOC system performance.

It’s further revealed that placing the receiver just under the OIRS (i.e., dsr = 40 m) causes the incidence angle at the photodetector to approach 90°. According to standard optical channel models based on Lambertian emission and projection laws, the received power is proportional to a  cos(Ψk,ls)  term, as expressed in (7). Hence,  as  the  incidence  angle  approaches  90°,  the  effective

OIRS-UWOC system efficiency versus number of water layers.

Influence of the link distance dsd on the OIRS-UWOC system.

received power tends toward zero, leading to a pronounced degradation in system performance.

Therefore, placing the receiver perpendicularly below the OIRS mirror array should be avoided, as this configuration effectively blinds the receiver and significantly weakens the communication link.

Fig. 6 plots the ABER for different OIRS array sizes. A clear decreasing trend in ABER is observed the reflecting surface incorporates more elements. Specifically, achieving an ABER level of 10−3 requires 43.75 dBm when only one element is employed, whereas this requirement drops substantially to 23.8 dBm, 17.8 dBm, 14.25 dBm, and 12 dBm when the OIRS comprises 100, 400, 900, and 1600 elements, respectively.

A pronounced performance improvement is achieved when increasing the number of elements up to 400, beyond which the incremental gains become progressively less significant. This saturation effect can be attributed to spatial correlation among adjacent elements, and phase control imperfections, which collectively   limit   the   effective   coherent   combining   at   the

ABER performance versus number of reflecting elements.

Comparison between OIRS-UWOC and NLoS-UWOC systems.

- receiver and result in marginal enhancements of the received signal quality.

- Fig. 7 compares the performance of the OIRS and NLoS configurations versus transmit power. The OIRS setup outperforms NLoS. This performance gain arises primarily from the avoidance of signal loss at the water–air interface, which is significant in the NLoS case due to reflection and refraction effects. Quantitatively, to achieve an ABER of 10−3, the OIRS-UWOC system requires approximately 17.8 dBm of transmit power, whereas the NLoS configuration demands around 57 dBm, yielding a power savings of roughly 39.2 dBm.

# Conclusion

The performance of the OIRS-UWOC system has been analyzed considering turbulence and attenuation effects. A closed-form formula for the ABER was obtained through the Laplace transform and Fox’s H-function. The analysis highlights the influence of fundamental system parameters, notably water type, source depth, communication range, and number of OIRS elements was thoroughly investigated. The results demonstrated that the integration of OIRS significantly improves the reliability of the system compared to the conventional NLoS-UWOC. Moreover, the results clearly showed a significant performance improvement with OIRS deployment. The analysis of optimal placement further revealed important design considerations: the OIRS should not be positioned directly beneath the transmitter or receiver, and neither node should be placed too close to the OIRS plane. These insights highlight practical guidelines for the efficient design of the OIRS-UWOC system.

##### References

[1]	C. A. Kerrache, C. Calafate, A. Lakas, and M. Lahby, Internet of unmanned Things (IoUT) and mission-based Networking. Springer, Cham, 2023.

[2]	A. A. El-Banna, and K. Wu, Introduction to underwater communication and IoUT networks. in: machine learning modeling for IoUT networks. Springer Briefs in Computer Science. Springer, Cham, 2021.

[3]	H. Haas, M. S. Islim, C. Chen, and H. Abumarshoud, An introduction to optical wireless mobile communication. Norwood, MA, USA: Artech House, 2021.

[4]	H. Kaushal and G. Kaddoum, “Underwater optical wireless communication,” IEEE Access, vol. 4, pp. 1518–1547, 2016.

[5]	R. P. Naik and W. Y. Chung, “Evaluation of reconfigurable intelligent surface-assisted underwater wireless optical communication system,” IEEE/OSA Journal of Lightwave Technology, vol. 40, no. 13, pp. 4257–4267, 2022.

[6]	Y. Ata, H. Abumarshoud, L. Bariah, S. Muhaidat and M.A. Imran, “Intelligent reflecting surfaces for under water visible light communications,” IEEE Photonics Journal, vol. 15, no. 1, pp. 1–10, 2023.

[7]	R. Salam, A. Srivastava, V. A. Bohara and A. Ashok, “An optical intelligent reflecting surface-assisted underwater wireless communication system,” IEEE Open Journal of the Communications Society, vol. 4, pp.1774–1786, 2023.

[8]	Y. Ata, M. C. Gökçe and Y. Baykal, “Intelligent reflecting surface aided vehicular optical wireless communication systems using higher-order mode in underwater channel,” IEEE Transactions on Vehicular Technology, vol. 73, no. 8, pp. 11196–11208, 2024.

[9]	Q. Zhang, D. W. Yue and X. Y. Xu, “Performance analysis of reconfigurable intelligent surface-assisted underwater wireless optical communication systems,” IEEE Photonics Journal, vol. 16, no. 4, pp. 1–14, 2024.

[10]	A. Aissaoui, and L. Hacini, “Enhancing UWOC link performance using a hybrid OFDM/SAC-OCDMA system,” Optical and Quantum Electronics, vol. 56, 2024.

[11]	C. S. S. Shetty, R. P. Naik, U. S. Acharya, and W. Y. Chung, “Performance analysis of underwater vertical wireless optical communication system in the presence of weak turbulence, pointing errors and attenuation losses,” Optical and Quantum Electronics, vol. 55, no. 1, pp. 1–22, 2023.

[12]	P. N. Ramavath, S. A. Udupi, and P. Krishnan, “Co-operative RF-UWOC link performance over hyperbolic tangent log-normal distribution channel with pointing errors,” Optics Communications, vol. 469, 2020.

[13]	M.R. Biswal, T.S. Delwar, A. Siddique, P. Behera, and J. Y. Ryu, “Analysis and investigation of a novel underwater channel model for OWC under log-normal turbulent condition,” Photon. Netw. Commun., vol. 47, pp. 1–8, 2024.

[14]	S. Aboagye, T. M. N. Ngatched, O. A. Dobre, and A. R. Ndjiongue, “Intelligent reflecting surface-aided indoor visible light communication systems,” IEEE Communications Letters, vol. 25, no. 12, pp. 3913–3917, 2021.

[15]	O. S. Badarneh, M. K. Awad, S. Muhaidat and F. S. Almehmadi, “Performance analysis of intelligent reflecting surface-aided decode-and-forward UAV communication systems,” IEEE Systems Journal, vol. 17, no. 1, pp. 246–257, 2023.

[16]	Z. Rahman, N. V. Tailor, S. M. Zafaruddin and V. K. Chaubey, “Unified performance assessment of optical wireless communication over multi-layer underwater channels,” IEEE Photonics Journal, vol. 14, no. 5, pp.1–14, 2022.

[17]	E. Zedini, et al., “Unified statistical channel model for turbulence induced fading in underwater wireless optical communication systems,” IEEE Trans. Commun., vol. 67, no. 4, pp. 2893-–2907, 2019.

[18]	A. Mathai, R. K. Saxena, and H. J. Haubold, The H-Function: Theory and Applications. New York, NY, USA: Springer, 2010.