Graphene-Based Narrow Band Absorber for Sensing Application

Aliakbar Dastranj
Electrical Engineering Department
Yasouj University
Yasouj, Iran
Email: dastranj@yu.ac.ir

Ling Shing Wong
Faculty of Health and Life Sciences INTI -IU University
Nilai, Malaysia;

Faculty of Nursing

Shinawatra University
Pathum Thani, Thailand
Email: lingshing.wong@newinti.edu.my 
Saman Heidari
Electrical Engineering Department
Yasouj University
Yasouj, Iran
Email: saman.heidari@stu.yu.ac.ir

Ayman Amer
Faculty of Engineering
Zarqa Univeristy
Zarqa, Jordan
Email: aamer@zu.edu.jo

Abstract—In this paper, a graphene-based narrow band absorber with sensing application in the far infrared band has been presented. The proposed structure has an absorption of 1 and also high sensitivity has been achieved by changing the refractive index of the surrounding medium. The effect of important geometrical parameters and the chemical potential and relaxation time of graphene on the performance of the designed absorber has been shown.

Keywords—absorber, graphene, plasmonic, narrow band, sensor.

# Introduction

Surface plasmons (SPs) can be defined as electromagnetic waves that propagate at the boundary between conductor and insulator. Local surface plasmons (LSPs) are the non-emission excitation of electrons on the surface of nanostructures due to the coupling of electromagnetic fields [1]. Surface plasmon polaritons (SPPs) are excited in the vicinity of insulator with thin metal films and are damped in the direction perpendicular to the surface. Excitation of LSPs and SPPs leads to enhancement of electromagnetic fields around thin metal films, gratings or nanostructures. The wavelengths in which LSP and SPP are excited, are sensitive to the geometrical parameters of the plasmonic structures [2-4].

The operation of graphene-based plasmonic sensors is based on the resonance of graphene surface plasmon polaritons (GSPPs), and changing the environment surrounding the sensor changes the operating wavelength of the sensor [5-6]. Sensors can be used to ensure human health in various industrial environments, which leads to improved human health by reducing health risks and significantly increasing life expectancy [7].

Graphene is a single-layer structure with a honeycomb arrangement of carbon atoms, and due to its properties, including tunability, having suitable properties in a wide frequency range, and high carrier velocity, it can be a good option for use in absorbers [8]. In order to better understand of graphene, can also be mentioned to its other properties, such as low loss and propagation capability of SPs [9-10]. Also, the ability to change the planar conductivity of graphene over a wide frequency range by changing the static electric field or chemical injection has attracted more attention to this material [11]. Given the importance of graphene, many absorbers have been designed and manufactured based on this material, including frequency-switchable absorbers based on thin graphene strips, graphene nanodisks, and even combinations of graphene wires with crossed gold wires [12-13]. In addition, graphene can be used in detectors [14], modulators [15], and sensors [16].

GSPPs can be excited in the terahertz and far-infrared frequency ranges. GSPP modes are stronger and have lower propagation losses relative to metal-based SPs. The high sensitivity of these sensors is due to the smaller resonance wavelength of graphene plasmons compared to the excitation wavelength, which leads to a stronger field and as a result greater interaction with the surrounding environment [17-18].

In this paper, graphene has been used to design a plasmonic absorber in the far-infrared frequency range, and a narrow band absorption with the value of 1 in the wavelength of 19.26 μm has been obtained. Also, the sensing application of the designed absorber has been investigated.

# Structure Geometry and Simulation Method

In Fig. 1, the three-dimensional view and top view of the designed absorber consisting of graphene ring and graphene quadrants are shown. Under the graphene layer, the layers of silicon dioxide (SiO2) layer and gold are located.

The conductivity of gold is  [19], and the permittivity of SiO2 is 2.2 [20]. Data related to graphene material is obtained according to the Kubo formula, which includes the intraband and interband transmission parts. In the far-infrared frequency range, interband transmission can be neglected. Therefore, considering intraband transition, the conductivity of graphene is obtained from the following equation [21]:

(1)

where e, , and  are the electron charge, angular frequency, and reduced Planck constant, respectively. Also, and  are the chemical potential and relaxation time of the graphene, respectively.

In this article, chemical potential and relaxation time are considered according to  and . In addition, the thickness of the graphene layer in the simulations has been considered to be 1 nm.

# Results and Discussion

A plane wave with electric field polarization x in the -z direction is incident perpendicular to the structure. The absorption of the structure is obtained using the equation A=1-R-T, where T and R are the transmission and the reflection of the absorber, respectively. Since the thickness of the gold layer is greater than the skin depth of gold in the far infrared wavelength range, therefore no wave passes through the structure and the transmission is zero.

The absorption curve of the designed device is plotted in Fig. 2, and absorption of 1 is observed in the wavelength of 19.26 µm.

The effect of the number of important geometrical parameters is investigated to obtain optimal values ​​and better understand the various structural components on the performance of the designed device. In Fig. 3, the absorption spectrum is shown for small (r1) and large (r2) radii of the graphene ring, where the optimal result is obtained for r1 = 60 nm and r2 = 95 nm. Also, in Fig. 4, the effect of the periodicity (p) on the absorption spectrum is shown. By increasing the period of the proposed device from 400 to 700 nm, the absorption rate of the structure gradually decreases due to the reduction of coupling between adjacent graphene rings.

(a)

(b)

- Fig. 1. (a) Three-dimensional view and (b) top view of a graphene-based narrow band absorber unit cell. t1= 800 nm, t2= 1300 nm, p= 400 nm, r1= 60 nm, r2= 95 nm, and r= 140 nm.

- Fig. 2. Absorption curve of the structure of Fig. 1 for chemical potential of  1eV.

In Fig. 5, the performance of the designed device is evaluated for chemical potentials (µc) 0.6, 0.8, and 1 eV. Since the carrier density increases with increasing µc, the absorption value increases. Also, with increasing µc, the operating wavelength of the designed device decreases, which of these feature is referred to as the tunability of the structure.

The absorption curve of the proposed device for different relaxation times (τ) of 0.2, 0.6, and 1 ps is shown in Fig. 6, which shows that with increasing relaxation time, the absorption value increases due to the increase in charges on the graphene surface.

A test material with a thickness of 20 nm is placed on top of the designed device, and the absorption curve of the designed device for different refractive indices of 1, 1.2, 1.4, and 1.6 is shown in Fig. 7. The sensitivity of the designed sensor is calculated according to formula , where  and  are the changes in the refractive index of the test material and operating wavelength, respectively [22].

By changing the refractive index according to 1 to 1.2, 1 to 1.4, and 1 to 1.6, the sensitivity values ​​are 1940, 1345, and 2323 nm/RIU, respectively. Therefore, the proposed structure has a maximum sensitivity value of 2323 nm/RIU. If the refractive index difference is less than 0.05, the changes are detectable by the sensor, but the sensitivity is low. For the refractive index of 1.05, the sensitivity is 1180 nm/RIU. Therefore, to have high sensitivity, the refractive index of the test medium must be greater than 1.05.

(a)

(b)

- Fig. 3. The influence of changing geometrical parameters of (a) r1 (b) r2 on the performance of the designed device.

- Fig. 4. The influence of changing geometrical parameter of p on the performance of the designed device.

- Fig. 5. The effect of changing the chemical potential (µc) of the graphene on the performance of the designed device.

- Fig. 6. The effect of changing the relaxation time () of the graphene on the performance of the designed device.

- Fig. 7. The effect of changing the refractive index of the test material on the performance of the designed device.

# Conclusion

In this study, a simple narrow band absorber with good sensing capability and usable in various applications has been presented. Changing the geometrical dimensions of the designed device to obtain optimal performance of the structure has been studied and investigated. The results showed that the best performance of the structure has been obtained for the chemical potential of 1 eV and the relaxation time of 1 ps, and also by changing the refractive index of the test material placed on top of the structure from 1 to 1.6, the maximum sensitivity value has been obtained.

##### References

M. Shabani and A. Mir, “Design and analysis of an ultra-broadband polarization-independent wide-angle plasmonic THz absorber,” in IEEE Journal of Quantum Electronics, vol. 57, no. 3, pp. 1-8, June 2021.

J. Liu et al., “Plasmonic nanocomposite for active dielectric metasurface terahertz absorbers,” in IEEE Transactions on Terahertz Science and Technology, vol. 13, no. 4, pp. 381-388, July 2023.

M. R. Rakhshani, “Narrowband plasmonic absorber using gold nanoparticle arrays for refractive index sensing,” in IEEE Sensors Journal, vol. 22, no. 5, pp. 4043-4050, 1 March 2022.

B. Zhang, R. Wang and X. Du, “Rapid and highly sensitive detection of sepsis inflammatory factors based on a plasmonic terahertz-metasurface biosensor,” in IEEE Transactions on Plasma Science, vol. 52, no. 4, pp. 1522-1530, April 2024.

A. Hamouleh-Alipour, M. Forouzeshfard, R. Baghbani and Z. Vafapour, “Blood hemoglobin concentration sensing by optical nano biosensor-based plasmonic metasurface: A feasibility study,” in IEEE Transactions on Nanotechnology, vol. 21, pp. 620-628, 2022.

P. Upender and A. Kumar, “Highly sensitive tunable dual-band THz refractive-based metamaterial sensor for biosensing applications,” in IEEE Transactions on Plasma Science, vol. 51, no. 10, pp. 3258-3264, Oct. 2023.

G. Xie, H. Attar, A. Alrosan, S. M. F. Abdelaliem, A. A. S. Alabdullah and M. Deif, “Enhanced diagnosing patients suspected of sarcoidosis using a hybrid support vector regression model with bald eagle and chimp optimizers,” PeerJ Computer Science, doi: 10.7717/peerj-cs.2455.

M. A. Jabbar, M. A. Naveed, M. Zubair, M. Q. Mehmood and Y. Massoud, “Wideband polarization insensitive tunable graphene-supported terahertz metamaterial absorber,” in IEEE Photonics Journal, vol. 15, no. 6, pp. 1-8, Dec. 2023.

R. Rahad, A. S. M. Mohsin, M. Belal Hossain Bhuian and M. M. Rahman, “Graphene-metamaterial based tunable broadband polarization insensitive absorber for terahertz antenna design,” in IEEE Access, vol. 12, pp. 48654-48667, 2024.

H. AbuOwida, S. Ibrahim Mohammad and A. Vasudevan, “Next-generation nanomedicine: the impact of graphene oxide and quantum dots on drug delivery,” Journal of Biomaterials Science, Polymer Edition, doi.org/10.1080/09205063.2025.2533475.

X. Du et al., “A broadband switchable metamaterial absorber/reflector based on multi-laps graphene sheets in the terahertz band,” in IEEE Photonics Journal, vol. 13, no. 5, pp. 1-8, Oct. 2021.

P. -Y. Chen and A. Alù, “Terahertz metamaterial devices based on graphene nanostructures,” in IEEE Transactions on Terahertz Science and Technology, vol. 3, no. 6, pp. 748-756, Nov. 2013.

F. B. Zarrabi, M. T. Islam, F. Kazemi and M. R. Rakhshani, “Miniaturization of the THz perfect absorber based on various spoof topologies,” in IEEE Transactions on Plasma Science, vol. 53, no. 7, pp. 1652-1659, July 2025.

S. B. Amlashi, M. Khalily, V. Singh, P. Xiao, J. D. Carey and R. Tafazolli, “Surface electromagnetic performance analysis of a graphene-based terahertz sensor using a novel spectroscopy technique,” in IEEE Journal on Selected Areas in Communications, vol. 39, no. 6, pp. 1797-1816, June 2021.

J. Crabb, X. Cantos-Roman, G. R. Aizin and J. M. Jornet, “Amplitude and frequency modulation with an on-chip graphene-based plasmonic terahertz nanogenerator,” in IEEE Transactions on Nanotechnology, vol. 21, pp. 539-546, 2022.

T. Chen, D. Liang and W. Jiang, “A tunable terahertz graphene metamaterial sensor based on dual polarized plasmon-induced transparency,” in IEEE Sensors Journal, vol. 22, no. 14, pp. 14084-14090, 15 July15, 2022.

M. Saqlain, M. A. Baqir and P. K. Choudhury, “MXene- and graphene-assisted THz metamaterial for cancer cells detection based on refractive index sensing,” in IEEE Transactions on Nanotechnology, vol. 23, pp. 652-657, 2024.

Y. Huang, S. Zhong, H. Yao and D. Cui, “Tunable terahertz plasmonic sensor based on graphene/insulator stacks,” in IEEE Photonics Journal, vol. 9, no. 1, pp. 1-10, Feb. 2017.

N. K. Panda, S. Mohapatra and S. Sahu, “Polarization insensitive graphene based tunable metasurface terahertz dual-band absorber,” in IEEE Transactions on Nanotechnology, vol. 24, pp. 34-41, 2025.

V. S. Yadav, B. K. Kaushik and A. Patnaik, “Broadband THz absorber for large inclination angle TE and TM waves,” in IEEE Photonics Journal, vol. 13, no. 5, pp. 1-7, Oct. 2021.

P. -Y. Chen, C. Argyropoulos and A. Alù, “Terahertz antenna phase shifters using integrally-gated graphene transmission-lines,” in IEEE Transactions on Antennas and Propagation, vol. 61, no. 4, pp. 1528-1537, April 2013.

M. Rezeg, A. Hlali and H. Zairi, “THz biomedical sensing for early cancer detection: metamaterial graphene biosensors with rotated split-ring resonators,” in IEEE Photonics Journal, vol. 16, no. 4, pp. 1-10, Aug. 2024.