Triple-Mode Wideband Filter Using Sixteenth-Mode SIW Technology

F.Z.SIABAH

Laboratory of Electromagnetism and Telecommunications

University Constantine 1,

Constantine 25000, Algeria

fzohra.siabah@gmail.com

Abstract— This work introduces triple-mode wideband filters based on a sixteenth-mode HMSIW resonator. The proposed design significantly reduces the size compared to conventional SIW cavities while preserving the resonant frequency. Enhanced selectivity and superior out-of-band performance make it well-suited for modern microwave and telecommunication systems.

Keywords —bandpass filter, circular cavity, sixteenth mode, substrate integrated waveguide (SIW)

# Introduction

Microwave filters are essential components in today’s wireless communication systems, ensuring proper frequency selection, mitigating interference, and reducing unwanted noise. With the emergence of fifth-generation (5G) networks and the ongoing evolution toward sixth-generation (6G) technologies, there is an increasing demand for compact, low-loss, and highly selective filters suitable for integration into high-frequency front-end modules.Over the past few years, substrate integrated waveguide (SIW) technology has gained significant attention as a promising solution in microwave and millimeter-wave design as a promising platform for realizing compact and high-performance microwave bandpass filters. Multi-mode SIW resonators, in particular, offer significant advantages in terms of size reduction, enhanced selectivity, and ease of integration with planar circuits. Unlike single-pole resonator filters, a single multi-mode SIW cavity can generate a wideband passband response while maintaining a reduced footprint, making it an efficient solution for next-generation communication systems. Dual-mode SIW filters have been extensively investigated, typically exploiting two degenerate modes within circular or square cavities. Mode excitation and coupling are commonly achieved through perturbations such as metallic vias, slots, or open-loop elements. For instance, Pradhan et al. In [1], the authors proposed a miniaturized dual-band SIW filter based on open-loop ring resonators, allowing both strong size reduction and high frequency discrimination. In another contribution, Rahman and Sarkar [2] developed a tunable dual-mode SIW cavity filter characterized by an extended upper stopband, making it particularly well-adapted for emerging 5G systems. Similarly, Praveena and Gunavathi [3] enhanced the filter selectivity by loading a dual-mode SIW structure with circular complementary split-ring resonators (CSRRs). Nevertheless, many existing dual-mode SIW filters remain constrained by a relatively narrow fractional bandwidth (FBW), typically below 10% which constrains their performance in broadband and next-generation communication systems.To overcome this limitation, recent studies have focused on triple-mode and higher-order multimode SIW configurations. Ammari et al. [4] proposed a wideband multi-mode SIW filter CSRRs and defected ground structures (DGS), achieving a fractional bandwidth approaching 50%. Pelluri and Kartikeyan [5] developed a compact triple-band half-mode SIW (HMSIW) filter that employs multiple cavity modes and a half-mode DGS structure for enhance both bandwidth and miniaturization. Feng et al. [6] demonstrated a modified rectangular SIW cavity supporting three distinct resonant modes, enabling compact triple-mode filters with enhanced out-of-band rejection. Higher-order multimode SIW resonators, including sixteenth-mode SIW (SMSIW) structures, have also been successfully implemented for advanced microwave applications [7]. These studies indicate that multimode SIW resonators—including triple-mode, hybrid HMSIW, and higher-order SMSIW structures—constitute a promising platform for wideband and ultra-wideband microwave filters in next-generation wireless communication systems.

In this work, a compact sixteenth-mode substrate integrated waveguide (SMSIW) bandpass filter is proposed to enhance the out-of-band performance of microwave filtering structures. The design employs a three-pole SMSIW configuration incorporating ground-plane apertures to improve frequency response, selectivity, and overall filter performance.

# Design Consideration

Figure 1 depicts the electric field distribution of the TM010 mode across different cavity configurations utilizing substrate integrated waveguide (SIW) technology. By successively dividing a standard SIW cavity with either perfect or fictitious magnetic walls, we obtain the HMSIW, QMSIW, EMSIW, and SMSIW structures. Each division reduces the cavity size by factors of 2, 4, 8, and 16, respectively, while keeping the resonant frequency nearly constant.

Fig.1 Simulated electric field distributions using fictitious magnetic walls for various SIW cavity configurations: (a) full-mode SIW, (b) HMSIW, (c) QMSIW, (d) EMSIW, and (e) SMSIW [8-9].

# Triple-Mode Wideband Filter

In this work, a triple-pole bandpass filter employing a sixteenth-mode substrate integrated waveguide (SMSIW) circular cavity with two ground slots is designed and analyzed. The structure is implemented on a Rogers RT/Duroid 5880 substrate (εr = 2.2, d= 0.787 mm, tan δ = 0.0009). The geometrical configuration of the proposed filter is presented in Fig.2, confirming the validity and effectiveness of the adopted design approach.

Fig. 2 Tree-pole bandpass filter with ground-plane apertures

The proposed filter comprises three cascaded SMSIW resonators coupled through finely optimized inter-resonator gaps to realize the desired passband response. Each resonator supports multiple electromagnetic modes that collectively shape a broad and well-defined passband. The incorporation of ground-plane apertures effectively suppresses unwanted higher-order modes and enhances out-of-band rejection, while preserving the compactness of the overall structure.The designed filter exhibits three distinct resonant frequencies at 1.8575 GHz, 2.2425 GHz, and 2.5050 GHz, corresponding to the three dominant poles. The optimized physical parameters of the structure are as follows:

strip width: Wstrip=2.4mm

Cavity thickness: t=17 um

Coupling gap: s=0.115mm

Cavity length: lf=24.5 mm

Slot width: Wf=0.2 mm

Slot separation: sf=0.5 mm.

The electric field distribution of the proposed filter structure is plotted in Fig.3. Through a figure we note that the maximum value electric field distribution is in the edge of the resonators

Fig. 3. E-field distribution at frequency (f=1.86GHz,f=2.24GHz,f=2.50GHz

The simulated transmission coefficient S12 (in dB) of the filter, shown in Fig. 4, confirms the presence of three distinct resonant modes, indicating successful triple-mode operation.

Fig.4 Simulated frequency response for Tree-pole bandpass filter.

The coupling coefficient kk between two adjacent SMSIW resonators can be determined from the following classical expression:

kk=fr12-fr22fr12+fr22                                                  (1)

where fr1 and fr2 represent the upper and lower –frequencies of the coupled resonators, respectively. The coupling coefficient kk quantifies the electromagnetic interaction between adjacent resonators and directly influences the filter bandwidth and selectivity. In practical implementations, this coupling can be accurately tuned by adjusting the gap s between neighboring SMSIW cavities, thereby enabling precise control over the bandwidth and overall frequency response. For g = 0.115, the coupling coefficients kk1 and kk2 are 0.215 and 0.066, respectively. Here, kk₁ denotes the coupling between cavities 1 and 2, while kk₂ corresponds to the coupling between cavities 1 and 3.

The simulated transmission coefficient S₁₂ (in dB) of the filter for different gap distances g is presented in Fig. 5. The results indicate a weaker electromagnetic interaction between adjacent resonators as the gap increases.

Fig.5 Effect of the Gap Parameter (s) on the S11 Response of the Triple-Mode Filter

The simulated transmission coefficient S12 of the proposed filter, shown in Fig. 6, is compared with the configuration without ground-plane apertures (Fig. 4). It is evident that the inclusion of these apertures significantly enhances stopband attenuation, effectively suppresses the undesired higher-order TM02 mode, and consequently improves both the frequency selectivity and overall filtering performance

Fig. 6 Tree-pole bandpass filter without ground-plane apertures

- TABLE I. Comparison Between Proposed Filter and Previously Reported SIW/SMSIW Dual and Triple-Mode Filters

| Ref | Mode Type | Freq (GHz) | Size (mm2) | FBW (%) | I-L (dB) | OBR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [10] | Dual-mode SIW | 2.4 | 48*46 | 7.5 | 1.6 | 22 |
| [11] | Dual-mode HMSIW | 3.5 | 30*30 | 22 | 1.4 | 25 |
| [12] | Triple-mode Circulair SIW | 2.45 | 52*52 | 18 | 1.8 | 32 |
|  | This work | 1.85- -2.50 | 40*50 | 22.4 | 0.85 | >30 |

Table I provides a detailed comparison between the proposed triple-mode SMSIW bandpass filter and several previously reported SIW- and SMSIW-based designs operating in similar frequency ranges. As shown, the sixteenth-mode cavity enables a substantial reduction in physical size.

In addition, the designed filter achieves three distinct resonant frequencies at 1.8575 GHz, 2.2425 GHz, and 2.5050 GHz, corresponding to the three dominant poles. The optimized geometry—including a strip width of 2.4 mm, a cavity thickness of 17 µm, and a coupling gap of 0.115 mm—enables a wide fractional bandwidth of approximately 22.4%, outperforming most previously published designs. The insertion loss is also notably low (0.85 dB), while the out-of-band rejection exceeds 30 dB, highlighting the superior selectivity achieved through the sixteenth-mode configuration. Overall, the comparison clearly demonstrates the enhanced miniaturization and improved filtering performance enabled by the proposed design.

# Conclusion

In this paper, a three-pole planar filter based on SMSIW resonators with ground-plane apertures is presented. The operating principle and design methodology are discussed in detail. This filter demonstrates low-loss characteristics across three different orders. The proposed approach provides an efficient and innovative method for designing passband periodic filters, highlighting its potential for advanced RF and microwave applications

# References

- [1]  N. C. Pradhan, S. Koziel, R. K. Barik, A. Pietrenko-Dąbrowska, and S. S. Karthikeyan, “Miniaturized Dual-Band SIW-Based Bandpass Filters Using Open-Loop Ring Resonators,” Electronics, vol. 12, no. 18,p.3974,2023.
[2] M. A. Rahman and P. Sarkar, “A Tunable Dual-Mode SIW Cavity Based Bandpass Filter with Wide Upper Stopband Characteristics,” Journal of Engineering Research and Sciences, vol. 2, no. 1, pp. 3–8, 2022.
[3]  N. Praveena and N. Gunavathi, “High Selectivity Dual-Mode Substrate Integrated Waveguide Bandpass Filter Loaded with Circular Complementary Split-Ring,” Progress in Electromagnetics Research Letters,vol.109,pp.17–23,2023.
[4] H. Ammari, F. Grine, M. L. Riabi, and M. T. Benhabiles, “Multi-Mode Substrate Integrated Waveguide Wideband Filter Design with Wide Stopband Rejection Using CSRRs and DGS,” Progress in Electromagnetics Research Letters, vol. 96, pp. 13–21, 2021.
[5]  S. Pelluri and M. V. Kartikeyan, “Compact Triple-Band Bandpass Filter Using Multi-Mode HMSIW Cavity and Half-Mode DGS,” International Journal of Microwave and Wireless Technologies, vol. 13,no.2,pp.167–174,2021.
[6]  Y. Feng, L. Zhang, X. Zhang, and Z. Chen, “Compact Triple-Mode Bandpass Filters Based on Single Modified Rectangular SIW Cavity,” Microwave and Optical Technology Letters, vol. 64, no. 10, pp. 699–706,2022.

- [7] F. Z. Siabah, F. Grine, M. T. Benhabiles, and M. L. Riabi, “Sixteenth‑Mode Substrate‑Integrated‑Waveguide (SMSIW) Resonator for Dielectric Characterization of Liquid Mixtures,” J. Microw., Optoelectron. Electromagn. Appl., vol. 19, no. 1, pp. 129–136, 2020.

- [8]  Huan Zhang, Yong-Chang Jiao, Gang Zhao and Chi Zhang, “Half-mode substrate integrated waveguide-based leaky-wave antenna loaded with meandered lines,” Electronics Letters, vol. 53, no. 17, pp. 1172–1174, Aug. 2017.

- [9]   Amit Ranjan Azad , Akhilesh Mohan“A compact sixteenth-mode substrate integrated waveguide bandpass filter with improved out-of-band performance,” Wiley; New York, NY, December 2016.

- [10] M. Rahman, M. K. Mandal, and S. K. Parui, “A compact dual-mode SIW bandpass filter using open-loop ring resonators,” IEEE Microwave and Wireless Components Letters, vol. 28, no. 4, pp. 310–312, Apr. 2018.

- [11] H. Liu, Y. Wang, and H. Zhang, “Dual-mode half-mode substrate integrated waveguide (HMSIW) bandpass filter with enhanced bandwidth,” IET Microwaves, Antennas & Propagation, vol. 13, no. 2, pp. 245–251, 2019.

- [12] J. Xu, W. Hong, K. Wu, and G. Ding, “Triple-mode circular SIW bandpass filter with high selectivity and compact size,” IEEE Transactions on Microwave Theory and Techniques, vol. 61, no. 3, pp. 1095–1103, Mar. 2013.