Review of Remote Sensing and Neural Networks in Solar Radiation Prediction for Smart Solar Power Plants

Mohammad Jafar Mokarram
School of Electronic Engineering and Intelligent Manufacturing
Anhui Xinhua University
Hefei, China
mjmokarram@axhu.edu.cn

Mohamed Hafez
Faculty of Engineering FEQS
INTI International University
Nilai, Malaysia 
Faculty of Management, Shinawatra University, Pathum Thani, Thailand, mohdahmed.hafez@newinti.edu.my
Marzieh Mokarram
Department of Geography, Faculty of Economics, Management and Social Sciences
Shiraz University
Shiraz, Iran
m.mokarram@shirazu.ac.ir

Hani Attar
Faculty of Engineering
Zarqa Univeristy
Jordan
Hattar@zu.edu.jo

Abstract—Solar power plant systems have complex nonlinear dynamics. This complexity is due to atmospheric fluctuations, solar radiation variations, and environmental uncertainties. Their accurate prediction with traditional methods is challenging. Remote sensing and deep neural networks such as Convolutional Neural Networks (CNNs), Recurrent Neural Networks (RNNs), and Long Short-Term Memory (LSTM) enable spatiotemporal data analysis. These methods offer superior performance in predicting solar radiation and irradiance for renewable energy generation. They are key in advanced sensor systems for designing, forecasting, maintaining, and controlling solar power plants. These methods have higher safety, reliability, and efficiency than classical approaches. The aim of this research is to review remote sensing and neural network technologies. The research explores their advantages such as high accuracy and generalizability, and their shortcomings compared to traditional methods for predicting solar radiation. Unlike other reviews, this research summarizes self-adaptive smart models. It presents simple but effective methods from remote sensing and neural network based sensor system. It also outlines the digital transformation to smart solar power plant with integrated technologies. The impact of these technologies on the renewable energy value chain is also evaluated.

Keywords—Remote Sensing, Neural Networks, Solar Radiation Prediction, Solar Power Plant, Machine Learning, Solar Irradiance, Renewable Energy.

# Introduction

In today’s world, energy crises and climate change are among the greatest challenges facing humanity. Renewable energies, especially solar energy, play a pivotal role in meeting sustainable energy needs. Solar energy is a clean and unlimited resource. It has great potential to reduce dependence on fossil fuels and reduce greenhouse gas emissions [1], [2]. It is regarded as a clean energy technology that significantly participates in energy development and developing countries progress [3], [4], [5]. The inherent fluctuations of solar radiation are affected by atmospheric factors such as clouds, dust, and seasonal changes. This complicates the accurate prediction of solar energy production [6]. Accurate solar radiation forecasting is essential for optimizing the performance of solar power plants. This forecasting plays a key role in energy integration into power grids, energy storage management, and economic planning. Without accurate forecasting, sudden fluctuations can lead to grid instability and increased operating costs [7]. Emerging technologies such as remote sensing and deep neural networks offer powerful tools to overcome these challenges. Remote sensing with satellite data allows monitoring of atmospheric patterns on a large scale. Neural networks such as Convolutional Neural Networks (CNNs), Recurrent Neural Networks (RNNs), and Long Short-Term Memory (LSTM) are capable of analyzing complex spatiotemporal data [1], [8], [9].

This combination increases the accuracy of the forecast and moves towards the creation of smart solar power plants. The importance of solar radiation forecasting in renewable energy is undeniable. Uncertainty in solar radiation can jeopardize economic opportunities. In areas with high solar potential, atmospheric fluctuations can reduce production [10]. Traditional forecasting methods, such as physical models based on the Navier-Stokes equations or statistical Autoregressive Integrated Moving Average (ARIMA) models, often fail in nonlinear and noisy data. These models rely on simplifying assumptions that are not valid in the real world, such as data stationarity or the absence of complex temporal dependencies. Their forecast errors can reach 20–30%. These errors are inadequate for operational planning of power plants. These limitations highlight the necessity of using data-driven methods. Machine learning and neural networks can extract hidden patterns from big data [11].

Remote sensing, as a key technology, has brought about a major revolution in solar radiation forecasting. This technology collects image and spectral data in real time with satellite sensors. The Geostationary Operational Environmental Satellite - R (GOES-R) series of satellites operated by NOAA provide high-resolution images. These images can be used for short-term forecasting [12]. Remote sensing data have a wide geographical coverage. They measure parameters such as cloud thickness, aerosol concentration, and atmospheric humidity. Integrating satellite data with deep learning models improves forecast accuracy

Figure 1. Combination of remote sensing and ANN for solar energy forecasting.

over ground-based models [13]. Remote sensing enables forecasting in remote areas where ground stations are limited.

Noise in satellite data due to atmospheric interference or resolution limitations requires advanced processing. Neural networks excel at this processing [14].

Deep neural networks have unique capabilities in processing complex data. CNN models are ideal for extracting spatial features from satellite images. RNN and LSTM are suitable for modeling temporal dependencies in solar radiation time series  [15]. These models do not require manual feature engineering and learn patterns automatically. Their advantages include high accuracy, generalizability, and the ability to handle large data sets. The shortcomings include computational complexity, the need for large training data, and the black-box nature. Explainable Artificial Intelligence (XAI) techniques are being developed to explain model decision making.

A review of the recent literature indicates the rapid growth of this field. Many studies in recent years have focused on the application of remote sensing and neural networks [16], [17], [18]. One review examined advanced distributed solar forecasting methods with remote sensing and deep learning, emphasizing their superiority in accuracy and generalizability [19]. Another study found neural network-based models to be superior for short-term forecasting, recommending LSTM and Gated Recurrent Units (GRU) models [20]. Integrating satellite data with ground-based measurements in hybrid models has also increased accuracy [21].

Studies show that multi-model models, combining machine learning and deep learning, perform better in long-term forecasting. The role of Artificial intelligence (AI) in solar power forecasting has also been examined in comprehensive reviews, emphasizing the improvement of forecasts [22], [23]. In practical applications, Supervisory Control and Data Acquisition (SCADA) systems play an important role in data collection.

These systems collect sensor data from solar panels and integrate it with machine learning models. A simple integration scheme can include APIs that connect neural network models with SCADA. This enables real-time forecasting. This integration increases sustainability and reduces costs. In large power plants, the use of this technology can improve efficiency by 10–15%. The digital transformation towards smart power plants with integrated technologies such as Internet of Things (IoT) and AI promises a bright future [24].

Figure 1 shows a combination of remote sensing and ANN for solar energy forecasting.

Given the importance of the topic, the objectives of this paper are to investigate neural network models in combination with remote sensing for solar energy monitoring and to solve the problems of accurate solar energy forecasting. Integrating remote sensing with neural networks solves current challenges and moves towards more sustainable energy.

# Machine Learning Technologies in Solar Radiation Forecasting for Renewable Energy Production

Solar power plants face significant challenges in accurately predicting energy production due to the nonlinear and variable nature of solar radiation. These variations are caused by atmospheric factors such as cloud cover, aerosols, and humidity. Traditional forecasting models, such as physical or statistical models, are ineffective against the complexity of real-world data. Machine learning technologies, especially deep neural networks, offer innovative solutions by analyzing big data and extracting complex patterns [25]. These technologies enable more accurate forecasting, higher stability, and better integration of solar energy into power grids.

- The Role of Machine Learning in Solar Radiation Forecasting

Machine learning has revolutionized solar radiation forecasting with models that extract hidden patterns from big, noisy data. Unlike traditional methods such as ARIMA models or physical equations based on fluid dynamics that rely on simplifying assumptions, machine learning models such as deep neural networks (DNNs) model complex nonlinear relationships without strong assumptions [26].

CNNs are effective for analyzing satellite imagery data, such as cloud cover images from GOES satellites. These networks extract spatial features such as cloud patterns and integrate them with temporal data [27]. RNNs and LSTM models are suitable for forecasting solar radiation time series, which contain long-term dependencies [9]. These models provide accurate predictions by processing SCADA data and remote sensing data. The advantages of machine learning include high accuracy, the ability to generalize to new data, and scalability to process big data. Challenges include the need for high-quality training data, computational complexity, and black-box nature. Training an LSTM model requires a large dataset that may not be available in areas with limited infrastructure [1]. To overcome this problem, techniques such as transfer learning or the use of synthetic data have been proposed.

- Integration of Remote Sensing with Machine Learning

Remote sensing data, such as satellite imagery and spectral data, play an important role in improving solar radiation forecasting. Satellites such as Himawari-8 or GOES-16 provide high-resolution data on atmospheric parameters such as cloud cover, aerosols, and surface temperature. These data are critical for both short- and long-term forecasting. When these data are integrated with machine learning models, multidimensional analysis is possible. A hybrid CNN-LSTM model can process satellite imagery to extract spatial features. The LSTM is then used to model temporal variations. Studies have shown that this approach improves forecast accuracy compared to models based on ground-based data [28].

The process of preparing remote sensing data involves various steps such as standardization, Principal Component Analysis (PCA), and feature extraction. These steps are essential to remove noise and improve data quality. Principal component analysis can reduce the dimensionality of spectral data while preserving key information [29], [30]. Sampling methods such as oversampling or under sampling are used to balance the data set. These methods are particularly useful in situations where data on rare weather events such as hurricanes are limited.

- Machine Learning Frameworks and Implementation

Machine learning frameworks such as TensorFlow, PyTorch, and Keras have made it easier to develop and deploy complex models. These frameworks support GPU parallel computing, reducing processing time. A CNN model on a GPU can process satellite images in fractions of a second, while traditional methods can take several minutes. Standards such as Cross-Industry Standard Process for Data Mining (CRISP-DM) provide a structured path for model development. This path includes data preparation, model training, validation, and deployment. The use of the Open Neural Network Exchange format allows for the transfer of trained models between different frameworks. This increases implementation flexibility and models can be deployed in different environments, including SCADA systems [31]. Self-adaptive models such as modified fuzzy neural networks (MFNN) can adapt to data changes and reduce overfitting problems [32].

- Proposed Pathway for Solar Radiation Forecasting

A proposed path for solar radiation forecasting involves the following steps:

1. Data collection: Using remote sensing and SCADA to create a comprehensive dataset.

2. Data preprocessing: Standardization, dimensionality reduction, and feature extraction using techniques such as PCA or Autoencoders.

3. Model selection: Selecting an appropriate model such as CNN-LSTM for short-term forecasting or GRU for long-term forecasting based on the data type and objective.

4. Training and validation: Training the model with training and validation datasets to avoid overfitting.

5. Deployment: Integrating the model into SCADA systems via APIs for real-time forecasting.

6. Continuous updating: Retraining the model with new data to maintain accuracy in changing atmospheric conditions.

- Challenges and Future Directions

Despite the advances, challenges remain, such as the need for advanced computing infrastructure, limited access to quality data, and lack of model transparency. Black-box models such as deep neural networks are challenging for power plant operators who need explainable decision-making. Explainable AI approaches can solve this problem. Developing lighter models that run on edge devices reduces computational costs and facilitates deployment in remote areas. Future directions include the development of multi-model models, the use of hybrid data to increase data diversity, and the integration of IoT technologies to collect real-time data. There is also an increasing focus on sustainability and reducing the carbon footprint of machine learning computational processes, especially in training large models.

# Machine Learning Methods in Solar Radiation Forecasting

Machine learning is a key tool in solar radiation forecasting for renewable energy generation. This technology provides more accurate solar radiation forecasting by analyzing big data and extracting complex patterns. Machine learning methods are divided into two categories: neural network (NN)-based methods and non-network methods. Each method has its own characteristics, advantages, and limitations. Performance evaluation criteria, implementation frameworks, and data sources are also reviewed.

## Classification of Machine Learning Methods

Machine learning methods include a variety of algorithms. These algorithms are divided into two groups: neural network-based and non-network-based. These methods are used for short-term and long-term forecasting and for analyzing spatial-temporal data.

A.1. Non-Neural Network Methods

Non-neural network methods include algorithms like PCA, Random Forest (RF), Support Vector Machine (SVM), and Decision Tree (DT). These methods are suitable for simpler tasks or when data requires significant preprocessing.

- Principal Component Analysis (PCA): PCA is a technique that reduces the dimensionality of high-dimensional data while preserving key information. The PCA formula is Z = X ×W, where X is the input data matrix, W is the matrix of eigenvectors, and Z is the reduced data. PCA is highly effective for reducing noise in remote sensing data, such as satellite imagery.

- Random Forest (RF): This method uses a collection of decision trees that operate in parallel. RF improves accuracy by averaging the predictions of individual trees. The RF prediction formula is yhat = (1/T) × (h1(x) + h2(x) + ... + hT(x)), where T is the number of trees and ht(x) is the prediction of the t-th tree. RF is suitable for solar radiation forecasting under varying atmospheric conditions.

- Support Vector Machine (SVM): SVM classifies or performs regression by finding an optimal hyperplane in a high-dimensional space. The SVM regression objective function is min (1/2) × ||w||2 + C A× (ξ1 + ξ2 + ... + ξn), where w is the weight vector, C is the regularization parameter, and ξi are the permissible errors. SVM performs well in short-term forecasting with limited data.

- Decision Tree (DT): This method divides data into a tree structure using decision rules. DT is suitable for initial analysis of solar data due to its simplicity and interpretability, but it may lead to overfitting with complex data.

A.2. Neural Network-Based Methods

Neural networks are highly effective for solar radiation forecasting because they can learn complex and automated features. These methods include:

- Artificial Neural Networks (ANN): ANNs consist of layers of neurons that process data using activation functions (e.g., ReLU or Sigmoid). The output of a neuron is given by y = f(w1x1 + w2x2 + ... + wnxn + b), where f is the activation function, wi are weights, xi are inputs, and b is the bias. ANNs are used for general solar radiation forecasting.

- Convolutional Neural Networks (CNN): CNNs are ideal for processing image-based data, such as satellite imagery. They use convolutional filters to extract spatial features. The convolution operation is (I × K)(x, y) = I(x+m, y+n) × K(m, n), where I is the input image and K is the convolution kernel. CNNs are highly effective for analyzing cloud cover patterns.

- Recurrent Neural Networks (RNN) and LSTM: RNNs are suitable for modeling time series but may face vanishing gradient issues. LSTM models address this with input, forget, and output gates. The forget gate equation in LSTM is ft = σ(Wf × [h(t-1), xt] + bf), where σ is the sigmoid function, Wf are weights, h(t-1) is the previous state, and xt is the current input. LSTMs are well-suited for long-term solar radiation forecasting.

- Transformers: These models process spatial-temporal data using an attention mechanism. The attention formula is Attention (Q, K, V) = softmax((Q × KT) / √dk) × V, where Q, K, and V are query, key, and value matrices. Transformers are used for more complex forecasting tasks.

- Modified Fuzzy Neural Networks (MFNN): These models combine fuzzy logic and neural networks, designed for adaptability to complex tasks. MFNNs use modified particle swarm optimization (PSO) to automatically generate optimal architectures.

## Ensemble Methods

Ensemble methods improve prediction accuracy and stability by combining multiple models. These methods include:

- Bagging: This method reduces variance by creating multiple data samples and averaging predictions. Random Forest is an example of bagging.

- Boosting: In this method, models are trained sequentially, with each model focusing on the errors of the previous one. The weight update formula in gradient boosting is w(t+1) = wt - η × ∇L(wt), where η is the learning rate and ∇L is the gradient of the loss function. XGBoost is an example of boosting.

- Stacking: This method combines multiple models and trains a meta-model for final predictions.

## Performance Evaluation Metrics

Model performance is evaluated using the following metrics:

- Mean Squared Error (MSE): MSE = (1/n) × ( (y1 - yhat1)2 + (y2 - yhat2)2 + ... + (yn - yhatn)2 ), where yi is the actual value and y-hati is the predicted value.

- Root Mean Squared Error (RMSE): RMSE = √(MSE).

- Mean Absolute Error (MAE): MAE = (1/n) × ( |y1 - yhat1| + |y2 - yhat2| + ... + |yn - yhatn| ).

- F1-Score and AUC: Used for evaluating classification models, especially with imbalanced data.

Normalized metrics like nRMSE and nMAE allow model comparison across different datasets.

## Machine Learning Frameworks

TensorFlow, PyTorch, Keras, and Scikit-learn frameworks are powerful tools for implementing models. They support GPU parallel computing and simplify the training, validation, and deployment process. The ONNX format allows for the transfer of models between frameworks. Big data libraries such as Apache Spark are used for processing big data.

# conclusion

Machine learning and remote sensing have revolutionized solar energy. These technologies have improved radiation and power forecasting, optimized design, predictive maintenance, and intelligent control of power plants. Advanced models such as CNN, LSTM, and MFNN have increased forecast accuracy by processing remote sensing data. These technologies have overcome the limitations of traditional methods such as ARIMA and increased the stability of power grids. Metaheuristic algorithms such as BPFPA have enabled accurate simulation of solar cells by minimizing RMSE.

CNN and RL models have identified failures with high accuracy and reduced costs. Challenges such as computational complexity, the need for high-quality data, and lack of model transparency remain. XAI approaches and lighter models for edge devices can solve these problems. Future directions include hybrid data with Generative Adversarial Networks (GANs), IoT integration for real-time data, and a focus on computational sustainability. Machine learning and remote sensing play a key role in making solar power plants smarter by increasing accuracy, efficiency, and sustainability. These technologies help meet clean energy goals and pave the way for a more sustainable future. Future work can explore the integration of solar forecasting models with cross-sector energy management frameworks [33]. Additionally, advanced signal processing and waveform optimization techniques can be used to improve methods for monitoring atmospheric aerosols and dust using remote sensing technologies [34], [35].

##### References

[1]	M. Mokarram, J. Aghaei, M. J. Mokarram, G. P. Mendes, and B. Mohammadi‐Ivatloo, “Geographic information system‐based prediction of solar power plant production using deep neural networks,” IET Renewable Power Generation, vol. 17, no. 10, pp. 2663–2678, 2023.

[2]	M. Mokarram, M. J. Mokarram, and A. Najafi, “Thermal power plants pollution assessment based on deep neural networks, remote sensing, and GIS: A real case study in Iran,” Mar Pollut Bull, vol. 192, p. 115069, 2023.

[3]	S. Dhivya and S. Prakash, “Power Quality Assessment in Grid-Connected Solar PV Systems Using Deep Learning Techniques,” Journal of Applied Data Sciences, vol. 6, no. 2, pp. 1192–1208, 2025.

[4]	H. Attar, A. Alahmer, G. Borowski, and S. Alsaqoor, “Comprehensive review of advancements, challenges, design, and environmental impact in floating photovoltaic systems.,” Ecological Engineering & Environmental Technology (EEET), vol. 26, no. 2, 2025.

[5]	A. Amer et al., “Floating photovoltaics: assessing the potential, advantages, and challenges of harnessing solar energy on water bodies,” Journal of Ecological Engineering, vol. 24, no. 10, pp. 324–339, 2023.

[6]	R. Zhang, M. Feng, W. Zhang, S. Lu, and F. Wang, “Forecast of solar energy production-A deep learning approach,” in 2018 IEEE International Conference on Big Knowledge (ICBK), IEEE, 2018, pp. 73–82.

[7]	J. H. R. Enslin, “Network impacts of high penetration of photovoltaic solar power systems,” in IEEE PES General Meeting, IEEE, 2010, pp. 1–5.

[8]	N. Elizabeth Michael, M. Mishra, S. Hasan, and A. Al-Durra, “Short-term solar power predicting model based on multi-step CNN stacked LSTM technique,” Energies (Basel), vol. 15, no. 6, p. 2150, 2022.

[9]	M. J. Mokarram, R. Rashiditabar, M. Gitizadeh, and J. Aghaei, “Net-load forecasting of renewable energy systems using multi-input LSTM fuzzy and discrete wavelet transform,” Energy, vol. 275, p. 127425, 2023.

[10]	R. E. Dickinson, “Solar variability and the lower atmosphere,” Bull Am Meteorol Soc, vol. 56, no. 12, pp. 1240–1248, 1975.

[11]	T. Ahmad, R. Madonski, D. Zhang, C. Huang, and A. Mujeeb, “Data-driven probabilistic machine learning in sustainable smart energy/smart energy systems: Key developments, challenges, and future research opportunities in the context of smart grid paradigm,” Renewable and Sustainable Energy Reviews, vol. 160, p. 112128, May 2022, doi: 10.1016/J.RSER.2022.112128.

[12]	T. J. Schmit et al., “Geostationary Operational Environmental Satellite (GOES)-14 super rapid scan operations to prepare for GOES-R,” J Appl Remote Sens, vol. 7, no. 1, p. 73462, 2013.

[13]	J. Qin, H. Jiang, N. Lu, L. Yao, and C. Zhou, “Enhancing solar PV output forecast by integrating ground and satellite observations with deep learning,” Renewable and Sustainable Energy Reviews, vol. 167, p. 112680, 2022.

[14]	N. Andriyanov and A. Kim, “Development and Training of a Neural Network Filter for Satellite Images Processing,” The International Archives of the Photogrammetry, Remote Sensing and Spatial Information Sciences, vol. 48, pp. 9–14, 2024.

[15]	M. C. Sorkun, Ö. D. Incel, and C. Paoli, “Time series forecasting on multivariate solar radiation data using deep learning (LSTM),” Turkish Journal of Electrical Engineering and Computer Sciences, vol. 28, no. 1, pp. 211–223, 2020.

[16]	J. F. Mas and J. J. Flores, “The application of artificial neural networks to the analysis of remotely sensed data,” Int J Remote Sens, vol. 29, no. 3, pp. 617–663, 2008.

[17]	S. Ding, H. Lin, X. Deng, W. Yao, and J. Jin, “A hidden multiwing memristive neural network and its application in remote sensing data security,” Expert Syst Appl, vol. 277, p. 127168, 2025.

[18]	D. Duan, P. Liu, B. Hui, and F. Wen, “Brain-inspired online adaptation for remote sensing with spiking neural network,” IEEE Transactions on Geoscience and Remote Sensing, 2025.

[19]	M. Massaoudi, I. Chihi, H. Abu-Rub, S. S. Refaat, and F. S. Oueslati, “Convergence of photovoltaic power forecasting and deep learning: State-of-art review,” Ieee Access, vol. 9, pp. 136593–136615, 2021.

[20]	R. Marquez, H. T. C. Pedro, and C. F. M. Coimbra, “Hybrid solar forecasting method uses satellite imaging and ground telemetry as inputs to ANNs,” Solar Energy, vol. 92, pp. 176–188, 2013.

[21]	F. A. Suwadana et al., “Solar Irradiance Estimation in Tropical Regions Using Recurrent Neural Networks and WRF Models,” Energies (Basel), vol. 18, no. 4, p. 925, 2025.

[22]	F. S. M. Abdallah, M. N. Abdullah, I. Musirin, and A. M. Elshamy, “Intelligent solar panel monitoring system and shading detection using artificial neural networks,” Energy Reports, vol. 9, pp. 324–334, 2023.

[23]	A. T. Eseye, J. Zhang, and D. Zheng, “Short-term photovoltaic solar power forecasting using a hybrid Wavelet-PSO-SVM model based on SCADA and Meteorological information,” Renew Energy, vol. 118, pp. 357–367, 2018.

[24]	E. R. Osoro, “Integrating Digital Transformation and Technological Innovation for Sustainable Renewable Energy Systems: The Role of AI, IoT, and Blockchain in Enhancing Efficiency and Grid Stability,” Multidisciplinary Journal of Technical University of Mombasa, vol. 4, no. 1, pp. 81–92, 2025.

[25]	Y. Ledmaoui, A. El Maghraoui, M. El Aroussi, R. Saadane, A. Chebak, and A. Chehri, “Forecasting solar energy production: A comparative study of machine learning algorithms,” Energy Reports, vol. 10, pp. 1004–1012, 2023.

[26]	M. Pikus and J. Wąs, “Using Deep Neural Network Methods for Forecasting Energy Productivity Based on Comparison of Simulation and DNN Results for Central Poland—Swietokrzyskie Voivodeship,” Energies (Basel), vol. 16, no. 18, p. 6632, 2023.

[27]	M. Segal-Rozenhaimer, A. Li, K. Das, and V. Chirayath, “Cloud detection algorithm for multi-modal satellite imagery using convolutional neural-networks (CNN),” Remote Sens Environ, vol. 237, p. 111446, 2020.

[28]	N. Li, L. Zhao, and H. Li, “BDS multiple satellite clock offset parallel prediction based on multivariate CNN-LSTM model,” GPS Solutions, vol. 28, no. 4, p. 189, 2024.

[29]	M. Mokarram, S. R. A. Ronizi, and S. Negahban, “Optimizing biomass energy production in the southern region of Iran: A deterministic MCDM and machine learning approach in GIS,” Energy Policy, vol. 195, p. 114350, 2024.

[30]	M. J. Mokarram, M. Mokarram, M. Khosravi, and Y. Cui, “Optimizing Risk Control for Solar Farm Green Management Using DNI Estimation at the Edge through a GLE-SVR Learning Model,” Tsinghua Sci Technol, 2024.

[31]	R. Wiencek et al., “Cyber Resilient 5G Enabled Communication Framework for Virtual Power Systems,” in 2025 International Conference on Communication, Computing, Networking, and Control in Cyber-Physical Systems (CCNCPS), IEEE, 2025, pp. 224–231.

[32]	C. Zhang, Y. Yu, Y. Wang, and M. Zhou, “Takagi–Sugeno fuzzy neural network hysteresis modeling for magnetic shape memory alloy actuator based on modified bacteria foraging algorithm,” International Journal of Fuzzy Systems, vol. 22, no. 4, pp. 1314–1329, 2020.

[33]	N. Gholipoor et al., “A Review on the Cross-Sector Resource Management Framework for Electric Vehicles Integration: Challenges, Solutions, Key-Enabling Technologies, and Future Directions,” IEEE Open Journal of Intelligent Transportation Systems, vol. 6, pp. 1084–1120, 2025, doi: 10.1109/OJITS.2025.3593437.

[34]	Z. Asadsangabi, R. Mohseni, and S. Samadi, “Optimizing the OFDM radar waveform for LATT using WAF criteria in smart cities,” EURASIP Journal on Wireless Communications and Networking 2025 2025:1, vol. 2025, no. 1, pp. 90-, Oct. 2025, doi: 10.1186/S13638-025-02525-1.

[35]	F. Aghaei, M. Uysal, R. C. De Lamare, and L. T. N. Landau, “Vehicular MMW Communication: Channel Characterization Based on Ray Tracing Study,” Proceedings of the International Symposium on Wireless Communication Systems, 2024, doi: 10.1109/ISWCS61526.2024.10639124.